Yamaha 6388 VPDC

3C2h (R): Misc Output Register

bit 2-3  Select Master Clock

0  Use CLK0 as Master Clock (Usually 25.175 MHz)

1  Use CLK1 as Master Clock (Usually 28.322 MHz)

2  Use CLK2 as Master Clock (Usually external)

3  Use CLK3 as Master Clock (Usually the panel clock)

3d4h index 72h (R/W): Video Control

bit 0  Display on monochrome monitor/flat panel using

Hatching for G/H code 7 if set, Gray scale if not.

1  Enable Gray Scale/Hatching generator

2  Enable Tonal Gradation if set.

3  Select one of 2 16 word by 6 bit Color Palettes.

4  Enable 64 colors on 8 color panel if set.

5  This bit is output at the CON pin.

3d4h index 73h (R/W): Hatching Select

bit 0  Use Hatching for G/H code 8 if set, Gray Scale if not.

1  Use Hatching for G/H code 9 if set, Gray Scale if not.

2  Use Hatching for G/H code 10 if set, Gray Scale if not.

3  Use Hatching for G/H code 11 if set, Gray Scale if not.

4  Use Hatching for G/H code 12 if set, Gray Scale if not.

5  Use Hatching for G/H code 13 if set, Gray Scale if not.

6  Use Hatching for G/H code 14 if set, Gray Scale if not.

7  Use Hatching for G/H code 15 if set, Gray Scale if not.

3d4h index 74h (R/W): Vertical Adjust

bit 0-7  Number of non-used lines at the top of the screen.

For Dual Panel Single Drive use (Number of blank lines at

the top-1)*2.

For other types use (number of blank lines at the top)-1.

3d4h index 75h (R/W): Raster Scan Control

bit 0-1  Line Repeat Cycle.

0  5 line cycle

1  6 line cycle

2  7 line cycle

3  8 line cycle

2-4  Raster Scan Correction for Dual Panels

Set to LRC-(NLUS-NVA) mod LRC -1.

LRC  is 3d4h index 75h bit 0-1.

NLUS is 3d4h index 78h-79h  Number of Lines on Upper Screen.

NVA  is 3d4h index 74h      Number of Vertical adjustments.

Note: The Multi Raster Scan defined in this and the next register is

used if 3d4h index 9 bit 7 is clear.

3d4h index 76h (R/W): Raster Scan Data

bit 0-7  Raster Scan Data. A clear bit indicates that the next scanline

should be repeated, whereas a set bit increments the line counter.

The bits are rotated in a 5, 6, 7 or 8 bit cycle depending on

3d4h index 75h bit 0-1.

3d4h index 77h (R/W): LC Adjust

bit 0-4  Latch Clock Adjust. Number of dots times to add to

Start and End Blanking times, as plasma and EL

panels require strict timings.

3d4h index 78h (R/W): Vertical Screen Size H

bit 0  Bit 8 of the Vertical Screen Size.

The lower 8 bits are in 3d4h index 79h.

3d4h index 79h (R/W): Vertical Screen Size L

bit 0-7  Vertical Screen Size bit 0-7. Bit 8 is in 3d4h index 78h.

For full screen panels this value is the number of lines

on the display-1 typically 479.

For Dual Panel Single Drive types use (number of lines in

each panel-1)*2 typically 478.

For Dual Panel Dual Drive types use (number of lines in

each panel-1) typically 239.

3d4h index 7Ah (R/W): Horizontal Screen Size

bit 0-7  Horizontal Screen Size. For Dual Panel Dual Drive types

this is (number of dots per line)/4 -1 typically 159.

For other types this is (number of dots per line)/8 -1

typically 79.

3d4h index 7Bh (R/W): M Signal Pulse Width H

bit 0-1  Bits 8-9 of the M Signal C ontrol.

Lower 8 bits are in 3d4h index 7Ch.

3d4h index 7Ch (R/W): M Signal Pulse Width L

bit 0-7  Bits 0-7 of the M Signal Control.

Bits 8-9 are in 3d4h index 7Bh bits 0-1.

This sets the cycle time for reversing the polarity of

the AC conversion signal M which is transmitted to the

LCD panel.

This is a way of controlling the brightness of the display.

3d4h index 7Dh (R/W): Panel Control

bit 0-1  Selects Panel Screen type.

0  1 Screen Panel

1  2 Screen Single Drive type

2  2 Screen Dual Drive type.

2-3  Selects number of bits to transfer for each pixel.

0 = 1 bit serial, 1 = 4 bit parallel and 2 = 8 bit parallel.

4  If set the Shift Clock is output constantly, otherwise

only during display intervals.

6  If set FLM is output 2 cycles delayed from VSYNC,

if clear they are simultaneous.

7  If set the cursor blinks at a field cycle of 32, 16 if not.

3d4h index 7Eh (R/W): Stand By

bit 0  If set VPDC functions are stopped and Stand-by mode is

entered. Operation is resumed when this bit is cleared.

If DRAMs are used for Video RAM the content of display

memory will be lost in Stand-by mode.

1  If set writing of the Configuration Switches Register

(3d4h index 7Fh) from the CPU is enabled.

3d4h index 7Fh (R/W): Configuration Switches

bit 0-3  Display Type

4-5  Panel/CRT connection

0  VGA CRT (analog) connected

1  EGA CRT (digital) connected

2 or 3  Flat panel connected

6  SRAM is used if set, DRAM else

7  Scratch

3d4h index 80h..FFh (R/W): Color Map Registers.

bit 0-3  Color Map for color #((index#-80h)*2)

4-7  Color Map for color #((index#-80h)*2+1)

Note: index 80h contains color #0 and 1, index 81h color #2 and 3 etc.

Grey Scale/Hatching

The Yamaha 6388 VPDC can support both analog, digital and

various panel, plasma and LCD displays.

In order to display original color information on displays

with limited colors (monochrome, 8, 16 or 64 colors/shades)

some conversions must be made.

The VPDC has a Color Mapping Table (CMT) which converts

the 8bit color signal to 4 bits.

This 4 bit code is either used directly for output

or sent through a Grey Scale/Hatching (G/H) converter.

G/H Code   Gradation    Hatching

0         ----   OFF   ----

1         ----   1/3   ----

2         ----   2/3   ----

3         ----   ON    ----

4         ----   1/4   ----

5         ----   2/4   ----

6         ----   3/4   ----

7         1/5         1,3/4

8         2/5         2,0/4

9         3/5         2,4/4

10         4/5         1/3 Hatch

11         1/6         2/3 Hatch

12         5/6         3\1 Hatch

13         2/7         3\2 Hatch

14         3/7         2|3 Hatch

15         4/7         1/2 Hatch

ID Yamaha VGA:

if isport($3d4,$7c,$7f) then YAMAHA_6388