###

### static char sccs_id[] = @(#)patch.doc       1.46 1/13/95 16:33:04";

###

-----------------------------------------------------------------

FLOATING-POINT DIVISION WITH OPTIONAL  CHECKING  TO  ENSURE  FULL

RESULT PRECISION

-----------------------------------------------------------------

Questions pertaining to documents and code related to  the  Intel

FDIV software patch can be directed to software-support@intel.com

or FAXed to (408)765-5165 attention FDIV PATCH.

CONTENTS

--------

REVISION HISTORY

OBJECTIVE

BACKGROUND

PATCH PROCESS STRATEGY

THE CORE ALGORITHM

RECOMPILATION

ISVs - END-USER MODIFICATIONS

PATCH IMPLEMENTATION

IMPLEMENTATION VARIATIONS

LIBRARIES

TECHNICAL NOTES

Running the Patch on Processors Preceding the

Intel486(tm) Processor

Detecting the Floating-Point Unit

Mnemonic Interpretation

Scaling Factor

Scaling Exceptions

Precision Loss

FPU Status Word

RELATED PROOFS

Safety of the Logarithmic Instructions FYL2X and FYL2XP1

FYL2X

FYL2XP1

Identifying Problematic Divisors

TESTING AND VALIDATION

CYCLE COST CONSIDERATIONS

APPENDIX A Division with optional checking functions summary

APPENDIX B Division with optional checking files summary

APPENDIX C Patch code revision history

REVISION HISTORY

----------------

011395

------

Added revision history section to patch document.

Retitled the section "Running the Patch on an  Intel386(tm)  Pro-

cessor"  to  "Running  the  Patch  on  Processors  Preceding  the

Intel486(tm) Processor."  Added text indicating the need to check

for the presence of a floating-point unit.

Added Table  2,  Executions  times  of  FDIV  patch  with  memory

operands, under CYCLE COST CONSIDERATIONS.

Added APPENDIX C containing patch code revision history.

OBJECTIVE

---------

The following document describes an Intel-approved  software  ap-

proach  to  floating-point division that utilizes proved software

algorithms and existing hardware instructions.   Using  this  ap-

proach  overcomes the possibility of a reduction in precision due

to a floating-point  division  flaw  in  some  steppings  of  the

Pentium(tm) processor.

The objectives of this approved approach are to provide a  method

for floating-point division that

1.  Ensures floating-point division result precision on

all Intel386 processors and beyond, in all precision

modes.

2.  Has been optimized for efficient performance on

current and future Intel processors.  This

optimization has been accomplished through the

hand-coding of assembly routines that include such

techniques as the elimination of branching code and

the avoidance of CPU stalls.

A number of software patches have been proposed that may be suit-

ed to avoiding a potential division flaw.  Note that Intel's pro-

posed  software  workaround,  or  patch,  does  not  disable  the

floating-point  unit on susceptible Pentium(tm) processors.  Hand

coded and optimized assembly routines were developed that contin-

ue to utilize the hardware floating-point division operation with

additional operations executed only in the rare case that a given

division  is known to be susceptible to a floating-point division

flaw.

The software patch presented is intended to be implemented at the

compilation  and  software  development levels.  End-users should

use recompiled code where  available.   Recompiled  code  with  a

patch  such  as  the  one  that Intel is providing will allow the

fastest patched executable speed.  Patches that interrupt execut-

ables  to override floating-point division operations with alter-

nate solutions incur the added expense interrupts.  The disabling

of  the  floating-point  unit  as  a patch for the floating-point

division flaw will slow all floating-point calculations,  includ-

ing those such as FADD that are unaffected by the FDIV flaw.

The utilization of other patches, such as those that disable  the

floating-point unit, can be of use to those who must execute code

that has not been coded or recompiled  with  incorporation  of  a

software patch.

BACKGROUND

----------

Certain steppings of the Intel Pentium(tm) processors have  exhi-

bited  a  flaw in the floating-point unit that may result in some

loss of precision in division results.  This precision  loss  can

manifest itself in bit positions 13 and beyond of the mantissa of

a floating-point division result and may  occur  in  any  of  the

three  (single,  double, and extended) precisions, independent of

rounding mode.   The  floating-point  division  flaw  can  affect

floating-point  division  instructions  such as FDIV and FDIVR as

well as functions utilizing hardware  division  instructions  in-

cluding  FPTAN,  FPATAN, FPREM, and FPREM1.  Because the flaw af-

fects a maximum of 5 sparsely populated divisor value ranges  out

of  1024 possible ranges and particular combinations of operands,

precision is only affected in approximately 1 of 9  billion  ran-

domly fed floating-point division operations.

Intel is determined to provide a safeguard against floating-point

division  inaccuracies expediently and on all processors.  To ac-

complish these goals, a unique collaboration was  formed  between

Intel and experts in the industry.  Analysis and software patches

for the Pentium(tm) processor floating-point division  flaw  have

been  devised  at  Intel utilizing expert input from Cleve Moler,

Terje Mathisen, Tim Coe, and Peter Tang.  Coe has  been  able  to

precisely  simulate  the FDIV flaw and provide proofs of correct-

ness for  the  techniques  described  in  this  document.   Moler

developed a software adjustment technique.  He has implemented an

FDIV patch in MATLAB  and  is  currently  verifying  the  result.

Mathisen  devised  a table-driven check of floating-point divisor

values, wrote an initial version of software patch code, and  as-

sisted  Intel  with  instruction-level  optimization  in assembly

code.  Intel extended these techniques further to provide  imple-

mentation  flexibility to software developers and to minimize the

clock count of the floating-point division precision  correction.

The  resulting  workaround  can  be implemented by replacing each

floating-point division instruction with a macro that expands  in

line.

PATCH PROCESS STRATEGY

----------------------

To avoid slowing the execution of already correct  floating-point

division  operations, the software patch first asserts the possi-

bility of an imprecise result before executing a correction.

The recommended software solution involves a  code  expansion  of

each  FDIV-type  instruction into a macro that includes a call to

an error checking and adjustment routine, described in  more  de-

tail  later in this document.  The routine includes several steps

needed  to  eliminate  any  potential  precision  loss  from  the

Pentium(tm) floating-point division flaw, as follows.

1.  Test a global flag to indicate whether or not a

processor is flawed.  If the processor is not a

Pentium(tm) processor containing the floating-point

division flaw, a normal floating-point hardware

division is applied to the original operands.

Otherwise,

2.  Perform an operand range check for those processors

which contain the flaw.

a.  If the range test indicates that a divisor

is not in a susceptible numeric range,

return the result of a normal floating-point

hardware division applied to the original

operands.  Otherwise,

b.  If the range test indicates that a divisor

is in a susceptible numeric range,

1.  Perform a software adjustment of the

numerator and denominator.

2.  Execute a hardware division with the

adjusted operands.

3.  Return the full precision result of

the division.

The software patch should be generated and  executed  by  default

rather than under a specific option flag.  In particular, "blend-

ed code," code targeted towards multiple processors,  should  in-

corporate  the software fix.  This guarantees that even execution

of applications not optimized for the Pentium(tm) processor  will

be  protected  against  floating-point precision reduction should

they unexpectedly be executed on a Pentium(tm) processor with the

floating-point  division  flaw.   An option should be provided to

disable the patch during execution.

In order to accommodate the requests of implementors of the Intel

software  patch, several variations to the basic software correc-

tion  and  its  implementation  have  been  developed  by  Intel.

Developers  can  choose  a  suggested  option that is technically

correct  in  their  environment,  minimally  intrusive  to  their

current  production  schedules,  and  allows for the fastest tur-

naround.  If a developer requires a solution not  encompassed  by

the  existing patches, Intel can assist in developing appropriate

techniques.

THE CORE ALGORITHM

------------------

The core of the division with optional checking process is accom-

plished through several steps and should not be modified.

The core algorithm need only be executed  on  flawed  Pentium(tm)

processors.   The  existence  of a Pentium(tm) processor with the

floating-point division processor flaw is identified at run  time

by  executing a floating-point division instruction with operands

known to induce a loss of precision.

During the first part of the core algorithm,  a  range  check  is

performed.   Only  divisors in identifiable ranges indicate divi-

sion operations susceptible to a floating-point  division  result

precision loss.  An early proof indicated that a maximum of 5 out

of 16 ranges, or 31% of ranges, of divisor values  identify  sus-

ceptible  divisions.   Tim  Coe  and  Peter Tang have proved that

there are a maximum of 5 out of 1024  ranges  of  divisor  values

that  constitute  susceptible  divisions,  or a potential of less

than 1%.  Peter Tang independently verified a proof of 5  suscep-

tible  numeric  bands  out of 128.  That proof currently supports

the core algorithm as it was available at the start of patch code

testing and validation.

Consider this representation of a normal divisor.

+------+-----------+-----+--------+-------------------------+

| sign |    exp    |  1. |  1111  | 111 . . . . . . . . . . |

+------+-----------+-----+--------+-------------------------+

|   |(RANGE)                         |

|   +----------- mantissa -----------+

+-- zero if denormal

Figure 1.

RANGE refers to the 4 bits of the mantissa  seen  in  the  figure

above.   In order for a reduction in precision to possibly affect

a floating-point division result, these 4 bits must be  equal  to

the  decimal  values 1, 4, 7, 10, or 13.  Furthermore, the subse-

quent three bits must all be ones (i.e. equal to a 3-bit value of

7 decimal).

Consider the floating point number  14.999999.   This  number  is

known  to  be  a divisor susceptible to a reduced precision divi-

sion.  Its hexadecimal value is 416FFFF.  Its binary  representa-

tion can be seen in Figure 2.

+------+-----------+-----+--------+-------------------------+

|  0   | 1000 0010 |  1. |  1101  | 111 1111 1111 1111 1111 |

+------+-----------+-----+--------+-------------------------+

|   |(RANGE)                         |

|   +----------- mantissa -----------+

+-- hidden bit

zero if denormal

Figure 2.

The first part of the range-check algorithm tests to see  if  the

divisor  in  question is a denormal number.  If the divisor value

is found to be denormal, it is shifted 2^64 to the left  to  nor-

malize the value before continuing with the range check process.

Next, the three bits following RANGE are masked.  If any of those

bits   equals  zero,  the  core  algorithm  executes  a  hardware

floating-point division with the original operands,  then  exits.

Using  the  Figure  2  example, 14.999999 is not denormal and the

three bits following RANGE are all ones.   Therefore,  the  algo-

rithm continues.

Next, an efficient table-lookup scheme developed by  Terje  Math-

isen  is  employed to detect a divisor whose RANGE value is 1, 4,

7, 10, or 13.  A table is initialized with 16 elements,  as  seen

in  Figure 2.  Positions 1, 4, 7, 10, and 13 are set to one while

the remaining positions are set to zero.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0|1|0|0|1|0|0|1|0|0|1|0|0|1|0|0|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

element 0                             element 15

Figure 3.

When the RANGE value is used as an index into the table, a one or

zero  is returned.  If a zero is returned, no precision reduction

will occur in a division result, and the core algorithm  executes

a  hardware  floating-point  division with the original operands,

then exits.

In the Figure 2 example, the 4 RANGE bits equal 13.  Indexing  13

into  the  table  (table[13]) returns a value of one, so the core

algorithm is not yet complete.

At this point in the algorithm, it  is  known  that  the  divisor

falls  into  a  numeric  range  that  may  be  susceptible to the

Pentium(tm) processor floating-point division flaw.  Both numera-

tor  and  divisor  are  multiplied by 15/16, which means that the

floating-point division itself is  multiplied  by  1  ((15/16)  /

(15/16)).   The scaling factor of 15/16 shifts the operand values

of the floating-point division into ranges known to be immune  to

the  floating-point division flaw.  That is, 1 is shifted to 0, 4

to 3, 7 to 6, and so on.  A hardware floating-point division with

the scaled operands is performed and the algorithm exits.

In single and double precision modes, division results will  have

full precision and conform to IEEE standards.  In extended modes,

at most one bit of  precision  may  be  lost  due  to  the  extra

floating-point operations on the operands.

RECOMPILATION

-------------

The Intel software patch can optimally be implemented at the com-

pilation  level.  The workaround can be implemented in a compiler

by following the guidelines under the section entitled PATCH  IM-

PLEMENTATION.    Compilers  should  be  modified  to  generate  a

software patch in place of each  originally  generated  FDIV-type

instruction  as  described  later.  Source-level applications can

then be recompiled to incorporate the Intel workaround.

Though a compiler should generate patch code by default,  an  op-

tion  should be available to disable patch code generation during

compilation.  When such an option is specified, a compiler should

still  be  able  to  check for the presence of the floating-point

division flaw and issue a warning if present.

ISVs - END-USER MODIFICATIONS

-----------------------------

The presented software solution can be  validly  applied  at  the

source  level;  however,  modifying an application at the source-

code level in this way can be error-prone and  catalyze  problems

in  future generations of the application.  Implementation of the

division_with_checking routine directly within a source-level ap-

plication  is not straightforward as during assembly code genera-

tion of an application.  Scanning specifically for floating-point

division symbols in a C program is a complicated process, for ex-

ample.  Multiple divisions in a statement or  within  conditional

blocks  prevent  the automatic expansion of a macro in place of a

normal floating-point division expression.   The  software  patch

should  preferably be implemented at the compilation level, where

operations have been broken into single independent  instructions

and  immediately  succeeding  function  calls  can  therefore  be

tolerated.

It is recommended that ISVs  recompile  their  applications  with

compilers  modified to generate the FDIV-instruction patch expan-

sion.  This will guarantee that  application  users  receive  the

precision  available from processors not containing the floating-

point unit flaw with  minimal  impact  on  application  execution

speed.

Individuals with access to their own applications' low-level  as-

sembly  source  codes  can  incorporate  the  provided macros for

floating-point division with checking much as  a  compiler  would

before code output.

If during a particular run an individual is certain that  an  ap-

plication  will  not be affected by a reduction in floating-point

division accuracy,  the  fdiv_chk_flag  global  variable  can  be

turned off to maximize execution speed of that application.

PATCH IMPLEMENTATION

--------------------

A   summary   of   some   precision   checking   and    restoring

(division_with_checking)  functions  is  provided  in Appendix A.

Refer to appropriate assembly source files for the  actual  Intel

division_with_checking  routines.   Throughout  the  remainder of

this document, the token division_with_checking is used to  indi-

cate  any  of the floating-point division patch routines provided

by Intel.

The preferred method of retaining precision involves compiler in-

clusion  of  a conditionally executed division_with_checking rou-

tine at each FDIV instruction originally generated.  This process

can  be expanded by compilers at a very late compilation phase so

as to preserve compiler state and minimize the  changes  required

to existing compilers.  Assembly code developers can directly in-

corporate calls to the patch code in their routines.

This late-phase design simplifies the implementation process  for

compiler  vendors  by  avoiding the disruption of the quality as-

surance process that would be caused  by  mid-compiler  modifica-

tions.   In addition, compiler optimization will proceed normally

since a call will not be inserted in place of an FDIV  in  inter-

mediate  code.  Insertion of a call during earlier phases of com-

pilation  could  potentially   turn   off   optimization   around

floating-point divisions.

In order to incur minimal cycle cost, the  division_with_checking

routines  should  be called only during execution of applications

on individual processors  known  to  exhibit  the  floating-point

division  flaw.  To this end, the software patch process includes

the  use  of   conditional   code   enclosing   the   appropriate

division_with_checking routines.

In the assembly code example presented in this section, the first

operand  of  a two-operand instruction is the result destination.

Consider an FDIVR (reverse  division)  instruction.   The  format

used in section examples is

FDIVR DEST, SRC

DEST <- SRC / DEST

The actual instruction used in this section's example will be

opcode mnemonic

d8 fd fdivr st, st(5)

The top of the stack will hold the  result  of  a  floating-point

division  of  the fifth (zero-based) stack position by the top of

the stack.

At the affected FDIV-type instruction, macro expansion  and  gen-

eration  of  a  conditional  block  around  the fdivr instruction

should take place as seen below.

if (fdiv_chk_flag == 1) {

fdivr st, st(5)

}

else

division_with_checking FDIV expansion

The fdiv_chk_flag global variable is a three-state variable whose

value  is  set within the division_with_checking routines.  It is

initialized to 0 when declared, set to 1 on  processors  not  re-

quiring  floating-point division with checking, or set to -1 when

the executing Pentium(tm) processor exhibits  the  floating-point

division flaw.

The value of fdiv_chk_flag is set  by  the  function  fdiv_detect

during  the first invocation of a division_with_checking routine.

The fdiv_detect function stores a value to fdiv_chk_flag  depend-

ing  upon the status of the executing processor.  It also returns

the new value of fdiv_chk_flag  to  the  calling  function.   The

division_with_checking  routine  is  called when fdiv_chk_flag is

not equal to 1, and proceeds when fdiv_chk_flag is equal to -1.

cmp     fdiv_chk_flag, $1               ; compare global to 1

jne     L1                              ; if not 1 jump to L1

fdivr   st, st(5)                       ; else do hw fdivr

jmp     L2                              ; then jump to L2

L1:

division_with_checking FDIVR expansion  ; do fdivr w/checking

; during the first

; invocation, this

; routine else sets

; fdiv_chk_flag

L2:

Next, insert a call to an appropriate division_with_checking rou-

tine  (in  this  example, fdiv_r) within the conditional block in

addition to the original FDIV-type instruction.  The code for the

current example then resembles the sequence below.

cmp     fdiv_chk_flag, $1               ; compare global to 1

jne     L1                              ; if not 1 jump to L1

fdivr   st, st(5)                       ; else do hw fdivr

jmp     L2                              ; then jump to L2

L1:

call    fdiv_r

L2:

Note that the caller must save and restore  condition  codes  and

may  need  to  save and restore register contents around calls to

division_with_checking functions.  The eflags  register  contents

are destroyed by the division_with_checking routines, and the eax

register contents might be overwritten as specified below.

When performing register-register divisions, as  in  the  current

example,  the  eax  register is used to convey information to the

division_with_checking procedure to be executed.  Therefore,  the

caller  must  at times save and restore eax around invocations of

division_with_checking functions.  Alternatively, the eax  infor-

mation  can be pushed onto the stack with simple modifications to

the relevant division_with_checking routines.

For a division with register operands, one operand is taken  from

the  top  of  the  floating-point  stack,  and the stack position

number of the second operand needs to be recorded in the eax  re-

gister  along  with  additional  information  about  the intended

floating-point division as pictured in  Figure  4.   Only  the  6

lowest  bits of the eax register are used for this initialization

so potential operation in 16-bit mode is still valid.

-----------------------------------------------------------------

5   4   3   2   1   0    Bit position

+---+---+---+---+---+---+

|   |   |   |   |   |   |

+---+---+---+---+---+---+

|       |   |   |   |

+-------+   |   |   |

Indicates   |   |   +-- pop (DIVP)

stack       |   +-- reverse (DIVR)

position    +-- True - result is at ST(bits 3 to 5)

False - result is at ST(0)

Figure 4.

Register Initialization of eax

for register-register FDIV patch

-----------------------------------------------------------------

Alternatively,  when  floating-point  divisions  involve   memory

operands,  the  associated division_with_checking routines expect

that memory operands have been pushed onto the top  of  the  user

stack.   This  process  avoids prefix overrides.  Push memory in-

structions can be used to accomplish the memory operand setup  on

the  user stack, eliminating the need for additional register as-

signment.

In the given example, the last six bits of eax should be  set  to

101010  (decimal  42).  That is, refer to position 5 (101) in the

stack for the second operand, the top  of  stack  will  hold  the

result (0), reverse division is specified (1), and no pop will be

executed (0).

cmp     fdiv_chk_flag, $1               ; compare global to 1

jne     L1                              ; if not 1 jump to L1

fdivr   st, st(5)                       ; else do hw fdivr

jmp     L2                              ; then jump to L2

L1:

push    eax                             ; save eax

mov     eax, $42                        ; load eax for fdiv_r

call    fdiv_r                          ; do div w/checking

fstp    result                          ; get the div result

pop     eax                             ; restore eax

L2:

The division_with_checking routines such  as  fdiv_r  return  the

division result on the floating-point stack.

In UNIX format, the destination and source operands are reversed.

Hence,  the  preceding  example would be translated to the subse-

quent code.

** cmp     $1, fdiv_chk_flag               ; compare global to 1

jne     L1                              ; if not 1 jump to L1

** fdivr   %st(5), %st                     ; else do hw fdivr

jmp     L2                              ; then jump to L2

L1:

** push    %eax                            ; save eax

** mov     $42, %eax                       ; load eax for fdiv_r

call    fdiv_r                          ; do div w/checking

fstp    result                          ; get the div result

** pop     %eax                            ; restore eax

L2:

Asterisks indicate instructions modified  during  translation  to

UNIX format.

IMPLEMENTATION VARIATIONS

-------------------------

The core algorithm that performs the divisions and assures  accu-

racy  should  not  be  modified.  There are subtle end cases that

must be accounted for to provide results equivalent to  the  FDIV

operation  executing  on  processors not containing the floating-

point division flaw.

The previous sections describe the  preferred  software  solution

for  overcoming the Pentium(tm) processor floating-point division

flaw.  This solution is likely to accommodate most  environments.

A  variation  of  the  preferred  resolution may be necessary, as

described in the following paragraphs.

If the fdiv_chk_flag global can be set  in  a  program's  startup

code, the checking routines can be modified to eliminate the set-

ting and testing of the fdiv_chk_flag global variable.  This  el-

iminates  one  mandatory call to a division_with_checking routine

and an additional compare within checking routines on  processors

that are not susceptible to the floating-point division flaw.

There may be cases where testing a global variable is not practi-

cal  and  the  first test of the software patch will be to see if

the divisor falls into a problematic range.

Intel has developed 16-bit DOS, 32-bit DOS, and UNIX versions  of

the  division_with_checking assembly routines.  The code operates

in extended-precision mode.  The control word is  saved  and  re-

stored  within  the  division_with_checking code.  If it is known

that the processor is always operating in 80-bit precision  mode,

the control word save and restore code can be deleted.

Performing the scaling and result adjustment  for  all  floating-

point  divisions falling within susceptible ranges without regard

to the presence of a floating-point division flaw on the  execut-

ing  processor  is never recommended as this needlessly increases

processing time.

LIBRARIES

---------

Code needs to be modified so that floating-point instructions are

replaced  with  floating-point  division macros that ensure full-

precision division results.  It is essential  that  libraries  as

well  as  hand-coded  and  compiler-generated  code be made safe.

FDIV may occur in many library routines, especially the hyperbol-

ic  routines.   Other instructions including transcendentals that

may be present in library code need to be addressed.   These  are

currently known to include FPTAN, FPATAN, FPREM, and FPREM1.  The

logarithmic instructions FYL2X and  FYL2XP1  are  safe  from  the

floating-point  division flaw as is proved in the TECHNICAL NOTES

section.

An implementation of FPTAN using a hardware division  instruction

as  well  as  a  64-bit software version of FPATAN are available.

Software implementations of  FPREM  and  FPREM1  have  also  been

developed.

TECHNICAL NOTES

---------------

Intel has developed 16-bit DOS, 32-bit DOS, and UNIX versions  of

the  division_with_checking assembly routines.  The code operates

in extended-precision mode and is designed for Intel386(tm)  pro-

cessors and beyond.

Hardware behavior is not identically mimicked in  the  FDIV  wor-

karound  code.   This  section includes explanations of technical

details and a summary of the differences between  straightforward

hardware division and hardware division within the context of the

proposed software workaround.

Running the Patch on Processors Preceding the  Intel486(tm)  Pro-

cessor

-----------------------------------------------------------------

The Intel patch code has been  specifically  written  to  run  on

Intel486(tm) processors and beyond.  It should be guaranteed that

the the fdiv_chk_flag global variable is set before attempting to

execute  any  workaround  code  on  processors  earlier  than the

Intel486(tm).

The fdiv_detect routine does a check for the Pentium(tm)  proces-

sor  floating-point  division flaw with a sample division and can

be run on all processors in the Intel Architecture family  having

floating-point units.  This routine initializes fdiv_chk_flag.

Prior to executing fdiv_detect, the presence of a  floating-point

unit  must  be  established.   This  cannot be established in the

fdiv_detect routine as such a check would require  the  incorrect

reinitialization of the floating-point unit when checking for the

FDIV flaw.

Detecting the Floating-Point Unit

---------------------------------

Since some operating  systems  provide  means  of  disabling  the

floating-point  unit,  applications need to be aware that the in-

formation they need is whether the OS has enabled the FPU, rather

than whether the FPU exists.

Old 16-bit binaries typically handled the absence of an FPU  with

built-in emulators.  Most 32-bit operating systems provide emula-

tion capability so applications do not need to provide their own.

Hence,  if a user requests that the operating system turn off the

floating-point unit on 32-bit operating  systems,  floating-point

operations  will be emulated by the 32-bit OS.  Alternatively, if

a user requests that the operating system turn off the  floating-

point  unit  on  16-bit operating system, floating-point instruc-

tions will be skipped.

16-bit applications should continue to use the FINIT sequence  to

detect  if the floating-point unit is present.  For compatibility

issues on older processors, the CPUID instruction should  not  be

used to check for an FPU.

For 32-bit applications where most environments  already  provide

FPU  functionality  by  default, it is not necessary for applica-

tions to test for the presence of the FPU explicitly.

Mnemonic Interpretation

-----------------------

Mnemonics,  opcodes,  and  their  descriptions  adhere   to   the

Pentium(tm) Processor User's Manual.  In particular, the mnemonic

FDIVRP ST(x), ST represents the opcode DE F0+x and  the  mnemonic

FDIVP  ST(x),  ST  represents  the opcode DE F8+x.  Note that the

UNIX assembler erroneously attaches each of  these  mnemonics  to

the other's opcode (e.g. FDIVRP ST(x), ST represents DE F8+x).

Scaling Factor

--------------

The scaling factor of 15/16  was  chosen  to  guarantee  that  an

operand  lying  within one of the five flaw-susceptible ranges of

numbers will be scaled to  a  safe  region.   This  guarantee  is

trivially  proven  by  testing  the  endpoints  of the five known

numeric bands.  Refer to Statistical Analysis of  Floating  Point

Flaw  In  the  Pentium(tm) Processor (1994) (Sharangpani, Barton)

for the boundaries of the potentially unsafe regions.

Scaling Exceptions

------------------

Because the scaling factor is less than one,  it  introduces  the

possibility  of  an underflow when the numerator is multiplied by

it.  If the result of the final division is to be either 32 or 64

bits, this can be addressed by performing the scaling in extended

precision.  Since extended precision has a  minimum  exponent  of

2^-16382,  no  single  or  double-precision input operand has the

possibility of becoming a denormal when multiplied.  If the scal-

ing  factor  were greater than one, a similar argument shows that

overflow is not possible.

Unfortunately, the possibility of underflow persists  for  80-bit

operations  with  numbers  having magnitude less than (16/15)*2^-

16382.  Masking the underflow exception while doing  the  scaling

avoids the trap that would ordinarily occur.  However, the under-

flow bit is sticky, and hence a spurious underflow would still be

reflected.

Precision Loss

--------------

o Hardware division within the context of the FDIV software patch

employs different algorithms than a simple hardware division.  In

order to avoid excessive performance degradation,  a  few  varia-

tions  in the resulting precision between the two division possi-

bilities may be observed.

o Newton-Raphson methods are typically less precise due to  final

roundings.  Because the precision-restoring algorithm in the FDIV

and FPTAN patch routines introduces two additional floating-point

operations to the computation of a division, the precision of the

operation is reduced by 1 ULP (unit of  least  precision).   This

means that with the given algorithm, an 80-bit precision division

result is reduced from 64 bits of mantissa precision to  63.   By

doing  all scaling in extended precision and then dividing in ei-

ther single or double-precision accuracy, results of full  preci-

sion are produced in single and double-precision modes.

o When applied to small denormal numerators, the FDIV patch  code

may  produce  slightly different results in the least significant

binary digits.  This potentially occurs  when  the  numerator  is

denormal,  and  hence has a reduced number of significant digits.

For example, when an extended precision denormal  has  6  leading

zeros,  that  number only has 58 significant digits.  When such a

number is used in a division, the result will only have 58 signi-

ficant digits.

o If the inputs to FDIV or FPTAN patch routines are  not  exactly

representable  as singles or doubles, the result may differ by up

to 1 ULP.  Exactly representable single and double operands  will

produce exact results.

o The FPATAN patch routine result may differ in  extended  preci-

sion  by  as  much  as 3 ULPs.  For single and double, the FPATAN

patch routine result precision may differ by as much as 1.5 ULPs.

FPU Status Word

---------------

o The assembly routines provided by Intel should  not  be  called

from  code with exceptions unmasked where the values of the flags

denormal, inexact, or underflow are utilized.

o Hardware division within the context of the FDIV software patch

employs different algorithms than a simple hardware division.  In

order to avoid excessive performance degradation,  a  few  varia-

tions  in  the resulting FPU status word between the two division

possibilities may be observed.

o The inexact flag after scaling and a hardware division may  not

be  the  same  as  a  hardware division of the original operands.

Sometimes using the original operands results in an  inexact  ex-

ception while using the scaled operands does not, and vice versa.

o The denormal bit may be set differently after  division  within

the context of any of the patch routines.

o The FDIV patch may set the  underflow  flag  for  divisions  by

denormals when underflow would not otherwise be set.

o FDIV and FPTAN patch  routines  may  set  C1  differently  when

called with precision control set to extended.

o FDIV and FPTAN patch routines may set C1 differently if the in-

put operands were not exactly representable as singles or doubles

and the precision control is set to single or double, respective-

ly.   Exactly  representable single and double operands will pro-

duce exact results.

o FPREM and FPREM1 patch routines may not  set  C0,  C1,  and  C3

identically  if  the  given  instruction  performs  an incomplete

reduction.

o The patch code for FPATAN may set the values of C0, C2, and  C3

differently  than the hardware instruction.  Similarly, the patch

for FPTAN may set C0 and C3 differently than the  hardware.   C0,

C2,  and  C3  are  marked undefined for these instructions in the

reference manual, so proper existing code should  not  rely  upon

specific values for them regardless.

RELATED PROOFS

--------------

Safety of the Logarithmic Instructions FYL2X and FYL2XP1

--------------------------------------------------------

Peter Tang has proved the immunity of FYL2X and FYL2XP1 from  the

Pentium(tm)  processor floating-point division flaw.  Proofs fol-

low.

FYL2X

-----

The table-driven polynomial-base algorithm for FYL2X employs  one

division for arguments x in the range 7/8 < x < 9/8 and one divi-

sion for arguments in the range |x - 1| >= 1/8.  That is, 0  <  x

<=  7/8  or  x  >=  9/8.  The two divisions are used for argument

transformation.  Division does not impact this algorithm.

For 7/8 < x < 9/8, the division is correct, and  therefore  FYL2X

is  unaffected  for input arguments in this range.  The reason is

that for x in this range, the  transformation  used  1+x  as  the

denominator.   This  transformation  is  quite standard.  The bit

pattern for 1+x in this range is either

2^(-1) * 1.0000????.... or 1.111?????.....

Both bit patterns are safe denominators.

For |x - 1| >= 1/8, the denominator has a bit pattern  of  2^m  *

1.b1  b2 b3 b4 b5 b6 b7 .... where (b6 b7) = (1 0) or (0 1).  The

reason is that the denominator is obtained by x+c where c is  ba-

sically the leading bits of x.  Precisely, for

x = 2^k * 1. b1 b2 b3 b4 b5 ? ? ? ? ? ? ...

we have

c = 2^k * 1. b1 b2 b3 b4 b5 1 0 0 0 .... 0 0 0

Thus,

x + c = 2^(k+1) * 1. b1 b2 b3 b4 b5 b6 b7 ? ? ?

where (b6 b7) is (1 0) or (0 1).  To be more explicit, x + c is

2^k *    1. b1 b2 b3 b4 b5  ?  ?  ?  ?  ?  ?  ...

+ 2^k *    1. b1 b2 b3 b4 b5  1  0  0  0  0  0  ...

---------------------------------------------------

2^k * 1 b1. b2 b3 b4 b5  0  ?  ?  ?  ?  ?  ?  ...

+ 2^k       .  0  0  0  0  0  1  ?  ?  ?  ?  ?  ...

---------------------------------------------------

2^k * 1 b1. b2 b3 b4 b5 b6 b7  ?  ?  ?  ?  ?

Where b6 and b7 cannot both be ones.  (b7 ==  1  implies  the  ?s

above are zeroes, making b6 = 0+0 = 0).

Since b5, b6, and b7 must all be ones when the  flaw  is  encoun-

tered, this range is also safe.

FYL2XP1

-------

The algorithm is very much the same as FYL2X, and the  impact  of

division on it is also the same as it is on FYL2X.

For |x| < 1/8, a division is used where 2+x is a  safe  denomina-

tor.   For  |x| >= 1/8, overwrite x by 1+x and again use x+c as a

denominator.

Identifying Problematic Divisors

--------------------------------

Tim Coe has proved the validity of checking the bit patterns  in-

dicating  divisors at risk described previously in this document.

Tim Coe and Peter Tang are currently  preparing  a  formal  proof

that  will  submitted for publication in the near future.  A main

thrust of the proof is to establish that the following two  digit

sequences and P-D table accesses are the only paths to addressing

the flawed P-D entries:

For cases 1, 4, 7, 10, and 13 ==>

Cycle | Q digit  | P-D entry        | Minimum magnitude

| selected | accessed         | of ignored partial

|          |                  | remainder

--------------------------------------------------------

B-3   | -1 or -2 | no restriction   | no restriction

|          |                  |

B-2   | -2       | maximum entry    | 125/512

|          | for -2 digit     |

|          |                  |

B-1   | +2       | flawed P-D entry | 14/64

|          | less 1/8         |

|          |                  |

B     | +2 ==> 0 | flawed P-D entry | 0

|          |                  |

For cases 1, 7, and 13 ==>

Cycle | Q digit  | P-D entry        | Minimum magnitude

| selected | accessed         | of ignored partial

|          |                  | remainder

--------------------------------------------------------

B-3   | -1 or -2 | no restriction   | no restriction

|          |                  |

B-2   | -1       | maximum entry    | 125/512

|          | for -1 digit     |

|          |                  |

B-1   | +2       | flawed P-D entry | 14/64

|          | less 1/8         |

|          |                  |

B     | +2 ==> 0 | flawed P-D entry | 0

|          |                  |

The partial remainder has the form:

P-D table <> ignored

address   <> portion

XXXX.XXXxxxxxxxxxxx...

xxxxxxxxxxx...

0 <= ignored < 1/4

Start from cycle B and work backwards.  Either establish algebra-

icly  that  alternatives  cannot occur or assume some alternative

can occur and derive a contradiction.  Progressing backwards, es-

tablish two ones, then three, then four, then finally six ones in

positions 2^(-5) to 2^(-10) in the divisor are  required  to  ad-

dress  the flawed P-D entry.  Use preliminary restrictions on the

divisor to establish earlier entries in  the  above  tables,  and

then  use  these  facts  to establish tighter restrictions on the

divisor.

TESTING AND VALIDATION

----------------------

Intel has performed multiple levels of testing and validation in-

cluding

1.  Core routines

2.  Compiler builds

3.  Random tests

The Intel FDIV software patch has been incorporated into  Intel's

own compiler and tested extensively for correctness.  In addition

to recompilation and execution of the entire production  compiler

test suite, specific division tests were designed using test vec-

tors containing  billions  of  random  division  operand  values.

These random division tests were recompiled with Intel's compiler

and all subsequently executed divisions completed without  error,

and  with full precision to the extent of the exceptions noted in

this document.  Compiler vendors  implementing  Intel's  software

patch  will perform additional testing and validation of the pro-

cedure.

CYCLE COST CONSIDERATIONS

-------------------------

Performance impact from the software patch will be minimal.   The

multiple  steps proposed in the preceding sections are optimal as

they ensure absolute resolution of a possible FDIV flaw and  pro-

vide  the best possible performance.  The cost on systems without

the flaw is insignificant.  On the  other  hand,  performing  the

range  test  at all times would waste processor cycles on proces-

sors that do not exhibit the floating-point division flaw.

Table 1 includes execution times of tests run on various  proces-

sors.   Software tests consisting of multiple divisions performed

with and without the register-register FDIV software  patch  were

compiled  using  a  recent  Intel Reference Compiler for UNIX and

timed on the systems  listed  in  the  table  columns.   Table  2

displays execution times of the FDIV patch using memory operands.

The testing and timing code included a loop like the following.

for (i=0; i<NTIMES; i++) {

for (j=0; j<NPAIRS; j++) {

quot1d += (float) pair[j].num / (float) 80[j].den;

}

}

It can be seen that the execution time of operations in  addition

to  divisions,  such  as  loads,  are part of the timing results.

Hence, Tables 1 and 2 show the relative performance of  division-

intensive  code  with and without the software patch, rather than

actual division cycle counts.  Also note that on the Intel486(tm)

processors,  Imprecise Divisions due to the Pentium(tm) processor

FDIV flaw do not exist.

+-----------+-----------+-----------+

| Hardware  |  Hardware Division    |

| Division  |  With Checking        |

+-----------+-----------+-----------+

| All       | Precise   | Imprecise |

| Divisions | Divisions | Divisions |

+----------------------+-----------+-----------+-----------+

| 50Mhz 486            | 41.0 secs | 41.0 secs | 39.6 secs.|

+----------------------+-----------+-----------+-----------+

| 60Mhz flawed Pentium | 14.4 secs | 25.2 secs | 33.0 secs |

+----------------------+-----------+-----------+-----------+

| 60Mhz Pentium        | 14.4 secs | 14.7 secs | 14.7 secs |

+----------------------+-----------+-----------+-----------+

| 90Mhz flawed Pentium | 11.7 secs | 20.3 secs | 26.6 secs |

+----------------------+-----------+-----------+-----------+

| 90Mhz Pentium        | 11.7 secs | 11.9 secs | 11.9 secs |

+----------------------+-----------+-----------+-----------+

Table 1.

Execution times of FDIV patch with register operands

+-----------+-----------+-----------+

| Hardware  |  Hardware Division    |

| Division  |  With Checking        |

+-----------+-----------+-----------+

| All       | Precise   | Imprecise |

| Divisions | Divisions | Divisions |

+----------------------+-----------+-----------+-----------+

| 50Mhz 486            | 30.8 secs | 32.9 secs | 32.9 secs.|

+----------------------+-----------+-----------+-----------+

| 60Mhz flawed Pentium | 12.1 secs | 34.5 secs | 42.3 secs |

+----------------------+-----------+-----------+-----------+

| 60Mhz Pentium        | 12.1 secs | 13.1 secs | 13.1 secs |

+----------------------+-----------+-----------+-----------+

| 90Mhz flawed Pentium |  9.7 secs | 27.6 secs | 33.9 secs |

+----------------------+-----------+-----------+-----------+

| 90Mhz Pentium        |  9.7 secs | 10.5 secs | 10.5 secs |

+----------------------+-----------+-----------+-----------+

Table 2.

Executions times of FDIV patch with memory operands

The effective maximum penalty on a Pentium(tm) CPU with the divi-

sion  flaw  is  determined  by  comparing  the  normal  cost of a

floating-point division operation with  the  FDIV  operation  and

correctness  check.  Even on flawed Pentium(tm) processors, divi-

sion inaccuracies occur extremely rarely; therefore,  it  can  be

assumed  that  the  extra cost of restoring precision will almost

never be incurred.

-----------------------------------------------------------------

APPENDIX A

----------

Division with optional checking functions summary

-----------------------------------------------------------------

fdiv_detect

fdiv_detect performs a check for a reduction in precision and in-

itializes  a  global variable depending upon the result.  A value

of 1 or -1 is also returned to indicate the result  as  described

in the PATCH IMPLEMENTATION section.

fdiv_chk (x,y), frem_chk (x,y), frem1_chk (x,y), fatan_chk (x,y)

fdiv_chk operates in all three  (single,  double,  and  extended)

precisions.   The  other  functions are always extended-precision

operations.  They leave a function result on the user stack.

fdiv_r

Register-register division - the routine is  parameterized  using

the six lowest bits of eax.

fdiv_m32, fdiv_m63, fdiv_m16i, fdiv_m32i

The fdiv_ functions  perform  divisions  with  stack  and  memory

operands.   The  memory operand must be copied to the user stack.

The copy alleviates the overhead  of  handling  prefixes  in  the

run-time  routines.   The  function name following the underscore

indicates the size of the memory operand.

fdivr_m32, fdivr_m64, fdivr_m16i, fdivr_m32i

The fdivr_ functions are similar to the fdiv_ functions but  per-

form reverse divisions.

APPENDIX B

----------

Division with optional checking files summary

-----------------------------------------------------------------

The following UNIX assembly files containing  code  to  implement

division  with  checking  are available.  Corresponding files are

available for DOS operating systems under the names  *16.asm  and

*32.asm in addition to the UNIX *u.s files listed.

patchu.s

FDIV/FDIVR instruction replacement routines to handle all flavors

of division.  For example, register-register operand division and

register-memory operand division.

fchku.s

User routines to do single, double, and extended precision  divi-

sions.

fpremu.s

FPREM/FPREM1 instruction replacement code.  Also user routines to

do single, double, and extended-precision remainder operations.

wrapu.s

FPATAN  instruction  replacement  code.   After  saving  of   the

floating-point environment, calls fatan_chk.

fpatanu.s

FPATAN replacement code to be used in library routines.  Contains

the core fatan_chk routine.

fptanu.s

FPTAN instruction replacement code.

APPENDIX C

----------

Patch code revision history

-----------------------------------------------------------------

011395

------

Since the first posting of the patch code the  following  changes

have been made:

Changes:

1. patch16.asm (version 1.10):

The frame pointer BP was not initialized properly prior to check-

ing  for a memory NaN operand.  This change affects the following

patch routines:

fdiv_m32

fdiv_m64

fdivr_m32

fdivr_m64

2. patch16.asm (version 1.10)

patch32.asm (version 1.2)

patchu.s (version 1.3):

The check for whether the FP stack was full has been  changed  to

check  simply  for  whether it is possible to load a value to the

top of the FP stack.  The previous test for a  full  stack  would

fail  if the FP stack became unbalanced.  This change affects the

following patch routines:

fdiv_m32

fdiv_m64

fdiv_m16i

fdiv_m32i

fdivr_m32

fdivr_m64

fdivr_m16i

fdivr_m32i

3. fchk16.asm (version 1.9)

fchk32.asm (version 1.3)

fchku.s (version 1.5):

A specific processor check has been removed from the  fdiv_detect

routine.    A   sample   division  is  performed  to  detect  the

Pentium(tm) processor floating-point division flaw.  Prior to ex-

ecuting fdiv_detect on processors preceding the Intel486(tm) pro-

cessor, the existence of a  floating-point  unit  must  be  esta-

blished.

-----------------------------------------------------

fchk32.asm:

-----------

;

; static char sccs_id[] = "@(#)fchk32.asm       1.3  1/13/95  10:07:22";

;

; This code is being published by Intel to users of the Pentium(tm)

; processor.  Recipients are authorized to copy, modify, compile, use and

; distribute the code.

;

; Intel makes no warranty of any kind with regard to this code, including

; but not limited to, implied warranties or merchantability and fitness for

; a particular purpose. Intel assumes no responsibility for any errors that

; may appear in this code.

;

; No patent licenses are granted, express or implied.

;

;

;  PRELIMINARY VERSION of the software patch for the floating

;  point divide.

;

;

.386

.387

DATA32  SEGMENT DWORD USE32 PUBLIC 'DATA'

fdiv_risc_table DB      0, 1, 0, 0, 4, 0, 0, 7, 0, 0, 10, 0, 0, 13, 0, 0

fdiv_scale_1    DD      03f700000h              ;0.9375

fdiv_scale_2    DD      03f880000h              ;1.0625

one_shl_63      DD      05f000000h

;

;  This is the flag that is tested by all the compiler branches

;  before the divide instruction.

;

;       0 = unknown whether running on a part with the flaw

;       1 = running on a part with no flaw

;       -1= running on a part with the flaw

;

public  _fdiv_chk_flag

_fdiv_chk_flag  DD      0h

;

;  The following are initializers for known values that cause

;  the divide problem.

;

test_dividend   DB      0,0,0,0c0h,07eh,01h,050h,041h    ; x = 4195835.0

test_divisor    DB      0,0,0,080h,0ffh,0ffh,047h,041h   ; y = 3145727.0

one             DB      0,0,0,0,0,0,0f0h,03fh

DATA32  ENDS

;

;  Stack variables for divide routines.

;

DENOM           EQU     0

NUMER           EQU     12

PATCH_CW        EQU     32

PREV_CW         EQU     28

STACK_SIZE      EQU     32

MAIN_DENOM      EQU     4

MAIN_NUMER      EQU     16

;

;  Stack frame locations for  C/C++.

;  Single real is pushed as 4 bytes, Double real is pushed as 8 bytes and

;  Long Double (extended) real is pushed as 16 bytes. Needs to be changed the

;  following definitions for different calling conventions.

;

;

; Definition for Single Real

;

SINGLE1         EQU     4 + STACK_SIZE

SINGLE2         EQU     8  + STACK_SIZE

;

; Definition for Double Real

;

DOUBLE1         EQU     4  + STACK_SIZE

DOUBLE2         EQU     12  + STACK_SIZE

;

; Definition for Long Double Real (Extended)

;

LDOUBLE1        EQU     4 + STACK_SIZE

LDOUBLE2        EQU     20  + STACK_SIZE

SRESULT         EQU     12  + STACK_SIZE

DRESULT         EQU     20 + STACK_SIZE

ONESMASK        EQU     0e000000h

CODE32  SEGMENT DWORD USE32 PUBLIC 'CODE'

CODE32  ENDS

DATA32  SEGMENT DWORD USE32 PUBLIC 'DATA'

DATA32  ENDS

CONST32 SEGMENT DWORD USE32 PUBLIC 'CONST'

CONST32 ENDS

BSS32   SEGMENT DWORD USE32 PUBLIC 'BSS'

BSS32   ENDS

DGROUP  GROUP CONST32, BSS32, DATA32

CODE32  SEGMENT   DWORD USE32 PUBLIC 'CODE'

assume cs:CODE32, ds:FLAT, es:FLAT, ss:nothing

; In this implementation the

; fdiv_main_routine is called,

; therefore all the stack frame

; locations are adjusted for the

; return pointer.

fdiv_main_routine PROC  NEAR

fld     tbyte ptr [esp+MAIN_NUMER]   ; load the numerator

fld     tbyte ptr [esp+MAIN_DENOM]      ; load the denominator

retry:

;  The following three lines test for denormals and zeros.

;  A denormal or zero has a 0 in the explicit digit to the left of the

;  binary point.  Since that bit is the high bit of the word, adding

;  it to itself will produce a carry if and only if the number is not

;  denormal or zero.

;

mov     eax, [esp+MAIN_DENOM+4] ; get mantissa bits 32-64

add     eax,eax                 ; shift the one's bit onto carry

jnc     denormal                ; if no carry, we're denormal

;  The following three lines test the three bits after the four bit

;  pattern (1,4,7,a,d).  If these three bits are not all one, then

;  the denominator cannot expose the flaw.  This condition is tested by

;  inverting the bits and testing that all are equal to zero afterward.

xor     eax, ONESMASK           ; invert the bits that must be ones

test    eax, ONESMASK           ; and make sure they are all ones

jz      scale_if_needed         ; if all are one scale numbers

fdivp   st(1), st               ; use of hardware is OK.

ret

;

;  Now we test the four bits for one of the five patterns.

;

scale_if_needed:

shr     eax, 28                 ; keep first 4 bits after point

cmp     byte ptr fdiv_risc_table[eax], 0        ; check for (1,4,7,a,d)

jnz     divide_scaled           ; are in potential problem area

fdivp   st(1), st               ; use of hardware is OK.

ret

divide_scaled:

mov     eax, [esp + MAIN_DENOM+8]       ; test denominator exponent

and     eax, 07fffh             ; if pseudodenormal ensure that only

jz      invalid_denom           ; invalid exception flag is set

cmp     eax, 07fffh             ; if NaN or infinity  ensure that only

je      invalid_denom           ; invalid exception flag is set

;

;  The following six lines turn off exceptions and set the

;  precision control to 80 bits.  The former is necessary to

;  force any traps to be taken at the divide instead of the scaling

;  code.  The latter is necessary in order to get full precision for

;  codes with incoming 32 and 64 bit precision settings.  If

;  it can be guaranteed that before reaching this point, the underflow

;  exception is masked and the precision control is at 80 bits, these

;  six lines can be omitted.

;

fnstcw  [esp+PREV_CW]           ; save caller's control word

mov     eax, [esp+PREV_CW]

or      eax, 033fh              ; mask exceptions, pc=80

and     eax, 0f3ffh             ; set rounding mode to nearest

mov     [esp+PATCH_CW], eax

fldcw   [esp+PATCH_CW]          ; mask exceptions & pc=80

;  The following lines check the numerator exponent before scaling.

;  This in order to prevent undeflow when scaling the numerator,

;  which will cause a denormal exception flag to be set when the

;  actual divide is preformed. This flag would not have been set

;  normally. If there is a risk of underflow, the scale factor is

;  17/16 instead of 15/16.

;

mov     eax, [esp+MAIN_NUMER+8] ; test numerator exponent

and     eax, 07fffh

cmp     eax, 00001h

je      small_numer

fmul    fdiv_scale_1            ; scale denominator by 15/16

fxch

fmul    fdiv_scale_1            ; scale numerator by 15/16

fxch

;

;  The next line restores the users control word.  If the incoming

;  control word had the underflow exception masked and precision

;  control set to 80 bits, this line can be omitted.

;

fldcw   [esp+PREV_CW]           ; restore caller's control word

fdivp   st(1), st               ; use of hardware is OK.

ret

small_numer:

fmul    fdiv_scale_2            ; scale denominator by 17/16

fxch

fmul    fdiv_scale_2            ; scale numerator by 17/16

fxch

;

;  The next line restores the users control word.  If the incoming

;  control word had the underflow exception masked and precision

;  control set to 80 bits, this line can be omitted.

;

fldcw   [esp+PREV_CW]           ; restore caller's control word

fdivp   st(1), st               ; use of hardware is OK.

ret

denormal:

mov     eax, [esp+MAIN_DENOM]   ; test for whole mantissa == 0

or      eax, [esp+MAIN_DENOM+4] ; test for whole mantissa == 0

jnz     denormal_divide_scaled  ; denominator is not zero

invalid_denom:                          ; zero or invalid denominator

fdivp   st(1), st               ; use of hardware is OK.

ret

denormal_divide_scaled:

mov     eax, [esp + MAIN_DENOM + 8]     ; get exponent

and     eax, 07fffh             ; check for zero exponent

jnz     invalid_denom           ;

;

;  The following six lines turn off exceptions and set the

;  precision control to 80 bits.  The former is necessary to

;  force any traps to be taken at the divide instead of the scaling

;  code.  The latter is necessary in order to get full precision for

;  codes with incoming 32 and 64 bit precision settings.  If

;  it can be guaranteed that before reaching this point, the underflow

;  exception is masked and the precision control is at 80 bits, these

;  six lines can be omitted.

;

fnstcw  [esp+PREV_CW]           ; save caller's control word

mov     eax, [esp+PREV_CW]

or      eax, 033fh              ; mask exceptions, pc=80

and     eax, 0f3ffh             ; set rounding mode to nearest

mov     [esp+PATCH_CW], eax

fldcw   [esp+PATCH_CW]          ; mask exceptions & pc=80

mov     eax, [esp + MAIN_NUMER +8]      ; test numerator exponent

and     eax, 07fffh             ; check for denormal numerator

je      denormal_numer

cmp     eax, 07fffh             ; NaN or infinity

je      invalid_numer

mov     eax, [esp + MAIN_NUMER + 4]     ; get bits 32..63 of mantissa

add     eax, eax                ; shift the first bit into carry

jnc     invalid_numer           ; if there is no carry, we have an

; invalid numer

jmp     numer_ok

denormal_numer:

mov     eax, [esp + MAIN_NUMER + 4]     ; get bits 32..63 of mantissa

add     eax, eax                ; shift the first bit into carry

jc      invalid_numer           ; if there is a carry, we have an

; invalid numer

numer_ok:

fxch

fstp    st                      ; pop numerator

fld     st                      ; make copy of denominator

fmul    dword ptr[one_shl_63]   ; make denominator not denormal

fstp    tbyte ptr [esp+MAIN_DENOM]

fld     tbyte ptr [esp+MAIN_NUMER]      ; load numerator

fxch                            ; restore proper order

fwait

;  The next line restores the users control word.  If the incoming

;  control word had the underflow exception masked and precision

;  control set to 80 bits, this line can be omitted.

;

fldcw   [esp+PREV_CW]           ; restore caller's control word

jmp     retry                   ; start the whole thing over

invalid_numer:

;

;  The next line restores the users control word.  If the incoming

;  control word had the underflow exception masked and precision

;  control set to 80 bits, this line can be omitted.

;

fldcw   [esp + PREV_CW]

fdivp   st(1), st               ; use of hardware is OK.

ret

fdiv_main_routine       ENDP

;

; int fdiv_detect(void)

;

public _FDIV_DETECT

_FDIV_DETECT PROC NEAR

fld     qword ptr [test_dividend]

fwait

fdiv    qword ptr [test_divisor] ; no_error = ((x - (x / y)*y) < 1.0

fwait

fmul    qword ptr [test_divisor]

fwait

fsubr   qword ptr [test_dividend]

fwait

fcomp   qword ptr [one]

fwait

fstsw   ax

fwait

and     eax, 0100h              ; 0: uninitialized

shr     eax, 7

sub     eax, 1

mov     _fdiv_chk_flag, eax

ret

_FDIV_DETECT ENDP

;

;  The calling convention is DIV_CHK(double,double,double*)

;

;  This routine assumes a calling convention of double real being pushed as

;  8 bytes.

;

public  _DIV_CHK

_DIV_CHK     PROC    NEAR

sub     esp, STACK_SIZE

fld     qword ptr [esp+DOUBLE1]

fstp    tbyte ptr [esp+NUMER]

fld     qword ptr [esp+DOUBLE2]

fstp    tbyte ptr [esp+DENOM]

call    fdiv_main_routine

mov     eax, [esp+DRESULT]

fstp    qword ptr [eax]

add     esp, STACK_SIZE

ret

_DIV_CHK     ENDP

;

;  The calling convention is FDIV_CHK(float,float,float*)

;

;  This routine assumes a calling convention of single real being pushed as

;  4 bytes.

;

public  _FDIV_CHK

_FDIV_CHK     PROC    NEAR

sub     esp, STACK_SIZE

fld     dword ptr [esp+SINGLE1]

fstp    tbyte ptr [esp+NUMER]

fld     dword ptr [esp+SINGLE2]

fstp    tbyte ptr [esp+DENOM]

call    fdiv_main_routine

mov     eax, [esp+SRESULT]

fstp    dword ptr [eax]

add     esp, STACK_SIZE

ret

_FDIV_CHK     ENDP

;

;  The calling convention is LDIV_CHK(long double, long double) and

;  returns a long double, ie, result is in th top of FP Stack (ST).

;

;  This routine assumes a calling convention of long double real being

;  pushed as  16 bytes.

;

public  _LDIV_CHK

_LDIV_CHK     PROC    NEAR

sub     esp, STACK_SIZE

fld     tbyte ptr [esp+LDOUBLE1]

fstp    tbyte ptr [esp+NUMER]

fld     tbyte ptr [esp+LDOUBLE2]

fstp    tbyte ptr [esp+DENOM]

call    fdiv_main_routine

add     esp, STACK_SIZE

ret

_LDIV_CHK     ENDP

CODE32  ENDS

end

fpatan32.asm:

-------------

; static char sccs_id[] = "@(#)fpatan32.asm     1.1 12/22/94 12:49:10";

;

; This code is being published by Intel to users of the Pentium(tm)

; processor.  Recipients are authorized to copy, modify, compile, use and

; distribute the code.

;

; Intel makes no warranty of any kind with regard to this code, including

; but not limited to, implied warranties or merchantability and fitness for

; a particular purpose. Intel assumes no responsibility for any errors that

; may appear in this code.

;

; No patent licenses are granted, express or implied.

;

;

.386

.387

CODE32   SEGMENT PARA PUBLIC USE32 'CODE'

CODE32  ENDS

CONST   SEGMENT DWORD PUBLIC USE32 'DATA'

CONST   ENDS

CONST2  SEGMENT DWORD PUBLIC USE32 'DATA'

CONST2  ENDS

DATA32   SEGMENT DWORD PUBLIC USE32 'DATA'

Y               EQU     0

X               EQU     12

PREV_CW         EQU     24

PATCH_CW        EQU     28

SPILL           EQU     32

STACK_SIZE      EQU     36

pos_1   DD   00000000H

DD   3ff00000H

neg_1   DD   00000000H

DD   0bff00000H

dispatch_table  DD      offset FLAT:label0

DD      offset FLAT:label1

DD      offset FLAT:label2

DD      offset FLAT:label3

DD      offset FLAT:label4

DD      offset FLAT:label5

DD      offset FLAT:label6

DD      offset FLAT:label7

;end dispatch table

pi      DB      35H

DB      0c2H

DD      0daa22168H

DD      4000c90fH

pi_by_2 DB      35H

DB      0c2H

DD      0daa22168H

DD      3fffc90fH

flt_sixteen DD  41800000H

one_by_sixteen  DD 3d800000H

B1      DW      0AAA8H

DD      0AAAAAAAAH

DD      0BFFDAAAAH

B2      DW      2D6EH

DD      0CCCCCCCCH

DD      3FFCCCCCH

B3      DW      4892H

DD      249241F9H

DD      0BFFC9249H

B4      DW      0C592H

DD      3897CDECH

DD      3FFBE38EH

B5      DW      5DDDH

DD      0C17BC162H

DD      0BFFBBA2DH

B6      DW      4854H

DD      77C7C78EH

DD      3FFB9C80H

atan_k_by_16    dd 000000000H, 000000000H, 000000000H, 000000000H

dd 067EF4E37H, 0FFAADDB9H, 000003FFAH, 000000000H

dd 0617B6E33H, 0FEADD4D5H, 000003FFBH, 000000000H

dd 072D81135H, 0BDCBDA5EH, 000003FFCH, 000000000H

dd 06406EB15H, 0FADBAFC9H, 000003FFCH, 000000000H

dd 03F5E5E6AH, 09B13B9B8H, 000003FFDH, 000000000H

dd 026F78474H, 0B7B0CA0FH, 000003FFDH, 000000000H

dd 0611FE5B6H, 0D327761EH, 000003FFDH, 000000000H

dd 00DDA7B45H, 0ED63382BH, 000003FFDH, 000000000H

dd 0D9867E2AH, 0832BF4A6H, 000003FFEH, 000000000H

dd 0F7F59F9BH, 08F005D5EH, 000003FFEH, 000000000H

dd 071BDDA20H, 09A2F80E6H, 000003FFEH, 000000000H

dd 034F70924H, 0A4BC7D19H, 000003FFEH, 000000000H

dd 0B4D8C080H, 0AEAC4C38H, 000003FFEH, 000000000H

dd 0C2319E74H, 0B8053E2BH, 000003FFEH, 000000000H

dd 0AC526641H, 0C0CE85B8H, 000003FFEH, 000000000H

dd 02168C235H, 0C90FDAA2H, 000003FFEH, 000000000H

DATA32  ENDS

BSS32   SEGMENT DWORD PUBLIC USE32 'BSS'

BSS32   ENDS

EXTRN   _fdiv_r:NEAR

DGROUP  GROUP CONST,CONST2,DATA32,BSS32

CODE32   SEGMENT PARA PUBLIC USE32 'CODE'

ASSUME CS:CODE32,DS:FLAT,ES:FLAT, SS:nothing

public fatan_chk

public _fatan_chk

fatan_chk       PROC NEAR

_fatan_chk:

push    eax

push    ecx

push    edx

sub     esp, STACK_SIZE

fstp    tbyte ptr [esp+X]       ; save X

fstp    tbyte ptr [esp+Y]       ; save Y

mov     ecx, [esp+Y+4]

add     ecx, ecx

jnc     hw_fpatan               ; unnormals (explicit 1 missing)

mov     eax, [esp+X+4]

add     eax, eax

jnc     hw_fpatan               ; unnormals (explicit 1 missing)

mov     ecx, [esp+Y+8]          ; save high part of Y

mov     eax, [esp+X+8]          ; save high part of Y

and     ecx, 7fffh              ; Ey = exponent Y

jz      hw_fpatan               ; Ey = 0

and     eax, 7fffh              ; Ex = exponent X

jz      hw_fpatan               ; Ex = 0

cmp     ecx, 7fffh              ; check if Ey = 0x7fffh

je      hw_fpatan

cmp     eax, 7fffh              ; check if Ex = 0x7fffh

je      hw_fpatan

fld     tbyte ptr [esp+X]       ; reload X

fabs                            ; |X| = u

fld     tbyte ptr [esp+Y]       ; reload Y

fabs                            ; |Y| = v

;  The following five lines turn off exceptions and set the

;  precision control to 80 bits.  The former is necessary to

;  force any traps to be taken at the divide instead of the scaling

;  code.  The latter is necessary in order to get full precision for

;  codes with incoming 32 and 64 bit precision settings.  If

;  it can be guaranteed that before reaching this point, the underflow

;  exception is masked and the precision control is at 80 bits, these

;  five lines can be omitted.

;

fnstcw  [PREV_CW+esp]           ; save caller's control word

mov     edx, [PREV_CW+esp]

or      edx, 033fh              ; mask exceptions, pc=80

and     edx, 0f3ffh

mov     [PATCH_CW+esp], edx

fldcw   [PATCH_CW+esp]          ; mask exceptions & pc=80

xor     edx, edx                ; initialize sflag = 0

fcom                            ; |Y| > |x|

push    eax

fnstsw  ax

sahf

pop     eax

jb      order_X_Y_ok

fxch

inc     edx                     ; sflag = 1

order_X_Y_ok:

push    eax

mov     eax, 0fh

call    _fdiv_r                 ; v/u = z

pop     eax

fld     dword ptr flt_sixteen   ; 16.0

fmul    st, st(1)               ; z*16.0

; Top of stack looks like k, z

fistp   dword ptr [SPILL+esp]   ; store k as int

mov     ecx, [SPILL+esp]

shl     ecx, 4

fild    dword ptr[SPILL+esp]

fmul    dword ptr one_by_sixteen; 1.0/16.0

; Top of stack looks like g, z

fld     st(1)                   ; duplicate g

fsub    st, st(1)               ; z-g = r

fxch

; Top of stack looks like g, r, z

fmulp   st(2), st               ; g*z

; Top of stack looks like r, g*z

fld     qword ptr pos_1         ; load 1.0

faddp   st(2), st               ; 1+g*z

; Top of stack looks like r, 1+g*z

push    eax

mov     eax, 0fh

call    _fdiv_r                 ; r/(1+g*z) = s

pop     eax

fld     st(0)                   ; duplicate s

fmul    st,st(1)                ; t = s*s

; Top of stack looks like t, s

fld     st(0)

fmul    st, st(1)

; Top of stack looks like t2, t, s

fld     st(0)

fmul    st, st(1)

fld     tbyte ptr B6

fld     tbyte ptr B5

; Top of stack looks like B5, B6, t4, t2, t, s

fxch

fmul    st, st(2)

fld     tbyte ptr B4

fxch    st(2)

fmul    st, st(3)

; Top of stack looks like B5t4, B6t4, B4, t4, t2, t, s

fld     tbyte ptr B3

fxch    st(2)

fmul    st, st(5)

; Top of stack looks like B6t6, B5t4, B3, B4, t4, t2, t, s

fxch    st(3)

fmulp   st(4), st

fld     tbyte ptr B2

; Top of stack looks like B2, B5t4, B3, B6t6, B4t4, t2, t, s

fxch    st(3)

faddp   st(4), st

mov     eax, [esp+X+8]

fld     tbyte ptr B1

fxch

shl     eax, 16

; Top of stack looks like B5t4, B1, B3, B2, even, t2, t, s

fmul    st, st(6)

fxch    st(2)

add     eax, eax

fmul    st, st(5)

; Top of stack looks like B3t2, B1, B5t5, B2, even, t2, t, s

fxch    st(3)

adc     edx, edx                ; |sflag|Sx|

fmulp   st(5), st

fxch    st(2)

mov     eax, [Y+8+esp]          ; save high part of Y

fmul    st, st(5)

; Top of stack looks like B3t3, B5t5, B1, even, B2t2, t, s

fxch    st(2)

shl     eax, 16

fmulp   st(5), st

; Top of stack looks like  B5t5, B3t3, even, B2t2, B1t, s

fxch    st(2)

faddp   st(3), st

add     eax, eax

faddp   st(1), st

adc     edx, edx                ; |sflag|Sx|Sy|

; Top of stack looks like  odd, even, B1t, s

faddp   st(2), st

faddp   st(1), st

fmul    st,st(1)                ; s*(odd+even)

faddp   st(1), st               ; poly

fld     tbyte ptr atan_k_by_16[ecx]     ; arctan[k;16]

faddp   st(1), st               ; w = poly + arctan(g)

jmp     dword ptr dispatch_table[edx*4]

label0:

fldcw   [esp+PREV_CW]

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

label1:

fchs

fldcw   [esp+PREV_CW]

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

label2:

fld     tbyte ptr pi

fsubrp  st(1), st               ; pi - w

fldcw   [esp+PREV_CW]

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

label3:

fld     tbyte ptr pi

fsubrp  st(1), st               ; pi - w

fchs                            ; - (pi - w)

fldcw   [esp+PREV_CW]

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

label4:

fld     tbyte ptr pi_by_2

fsubrp  st(1), st               ; pi/2 - w

fldcw   [esp+PREV_CW]

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

label5:

fld     tbyte ptr pi_by_2

fsubrp  st(1), st               ; pi/2 - w

fchs                            ; - (pi/2 - w)

fldcw   [esp+PREV_CW]

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

label6:

fld     tbyte ptr pi_by_2

faddp   st(1), st               ; pi/2 + w

fldcw   [esp+PREV_CW]

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

label7:

fld     tbyte ptr pi_by_2

faddp   st(1), st               ; pi/2 + w

fchs                            ; -(pi/2+w)

fldcw   [esp+PREV_CW]

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

hw_fpatan:

fld     tbyte ptr [esp+Y]       ; reload Y

fld     tbyte ptr [esp+X]       ; reload X

fpatan

add     esp, STACK_SIZE

pop     edx

pop     ecx

pop     eax

ret

fatan_chk       ENDP

CODE32  ENDS

END

fprem32.asm:

------------

; static char sccs_id[] = "@(#)fprem32.asm      1.1 12/22/94 12:49:11";

;

; This code is being published by Intel to users of the Pentium(tm)

; processor.  Recipients are authorized to copy, modify, compile, use and

; distribute the code.

;

; Intel makes no warranty of any kind with regard to this code, including

; but not limited to, implied warranties or merchantability and fitness for

; a particular purpose. Intel assumes no responsibility for any errors that

; may appear in this code.

;

; No patent licenses are granted, express or implied.

;

;

.386

.387

;

;  PRELIMINARY VERSION of the software patch for the floating

;  point remainder.

;

CHECKSW MACRO

ifdef   DEBUG

fnstsw  [fpsw]

fnstcw  [fpcw]

endif

ENDM

DATA32  SEGMENT DWORD USE32 PUBLIC 'DATA'

;

;  Stack variables for remainder routines.

;

FLT_SIZE        EQU     12

DENOM           EQU     0

DENOM_SAVE      EQU     DENOM + FLT_SIZE

NUMER           EQU     DENOM_SAVE + FLT_SIZE

PREV_CW         EQU     NUMER + FLT_SIZE

PATCH_CW        EQU     PREV_CW + 4

FPREM_SW        EQU     PATCH_CW + 4

STACK_SIZE      EQU     FPREM_SW + 4

RET_SIZE        EQU     4

PUSH_SIZE       EQU     4

MAIN_FUDGE      EQU     RET_SIZE + PUSH_SIZE + PUSH_SIZE + PUSH_SIZE

MAIN_DENOM              EQU     DENOM + MAIN_FUDGE

MAIN_DENOM_SAVE         EQU     DENOM_SAVE + MAIN_FUDGE

MAIN_NUMER              EQU     NUMER + MAIN_FUDGE

MAIN_PREV_CW            EQU     PREV_CW + MAIN_FUDGE

MAIN_PATCH_CW           EQU     PATCH_CW + MAIN_FUDGE

MAIN_FPREM_SW           EQU     FPREM_SW + MAIN_FUDGE

ONESMASK        EQU     700h

fprem_risc_table        DB      0, 1, 0, 0, 4, 0, 0, 7, 0, 0, 10, 0, 0, 13, 0, 0

fprem_scale             DB      0, 0, 0, 0, 0, 0, 0eeh, 03fh

one_shl_64              DB      0, 0, 0, 0, 0, 0, 0f0h, 043h

one_shr_64              DB      0, 0, 0, 0, 0, 0, 0f0h, 03bh

one                     DB      0, 0, 0, 0, 0, 0, 0f0h, 03fh

half                    DB      0, 0, 0, 0, 0, 0, 0e0h, 03fh

big_number              DB      0, 0, 0, 0, 0, 0, 0ffh, 0ffh, 0feh, 07fh

ifdef   DEBUG

public  fpcw

public  fpsw

fpcw    dw      0

fpsw    dw      0

endif

FPU_STATE       STRUC

CONTROL_WORD    DW      ?

reserved_1      DW      ?

STATUS_WORD     DD      ?

TAG_WORD        DW      ?

reserved_3      DW      ?

IP_OFFSET       DD      ?

CS_SLCT         DW      ?

OPCODE          DW      ?

DATA_OFFSET     DD      ?

OPERAND_SLCT    DW      ?

reserved_4      DW      ?

FPU_STATE       ENDS

ENV_SIZE        EQU     28

DATA32 ENDS

CODE32  SEGMENT DWORD USE32 PUBLIC 'CODE'

CODE32  ENDS

DATA32  SEGMENT DWORD USE32 PUBLIC 'DATA'

DATA32  ENDS

CONST32 SEGMENT DWORD USE32 PUBLIC 'CONST'

CONST32 ENDS

BSS32   SEGMENT DWORD USE32 PUBLIC 'BSS'

BSS32   ENDS

DGROUP  GROUP CONST32, BSS32, DATA32

CODE32  SEGMENT   DWORD USE32 PUBLIC 'CODE'

assume cs:CODE32, ds:FLAT, es:FLAT, ss:nothing

fprem_common    PROC    NEAR

push    eax

push    ebx

push    ecx

mov     eax, [MAIN_DENOM+6+esp] ; exponent and high 16 bits of mantissa

xor     eax, ONESMASK           ; invert bits that have to be one

test    eax, ONESMASK           ; check bits that have to be one

jnz     remainder_hardware_ok

shr     eax, 11

and     eax, 0fh

cmp     byte ptr fprem_risc_table[eax], 0     ; check for (1,4,7,a,d)

jz      remainder_hardware_ok

; The denominator has the bit pattern. Weed out the funny cases like NaNs

; before applying the software version. Our caller guarantees that the

; denominator is not a denormal. Here we check for:

;       denominator     inf, NaN, unnormal

;       numerator       inf, NaN, unnormal, denormal

mov     eax, [MAIN_DENOM+6+esp] ; exponent and high 16 bits of mantissa

and     eax, 07fff0000h         ; mask the exponent only

cmp     eax, 07fff0000h         ; check for INF or NaN

je      remainder_hardware_ok

mov     eax, [MAIN_NUMER+6+esp] ; exponent and high 16 bits of mantissa

and     eax, 07fff0000h         ; mask the exponent only

jz      remainder_hardware_ok   ; jif numerator denormal

cmp     eax, 07fff0000h         ; check for INF or NaN

je      remainder_hardware_ok

mov     eax, [esp + MAIN_NUMER + 4]     ; high mantissa bits - numerator

add     eax, eax                ; set carry if explicit bit set

jnz     remainder_hardware_ok   ; jmp if numerator is unnormal

mov     eax, [esp + MAIN_DENOM + 4] ; high mantissa bits - denominator

add     eax, eax                ; set carry if explicit bit set

jnz     remainder_hardware_ok   ; jmp if denominator is unnormal

rem_patch:

mov     eax, [MAIN_DENOM+8+esp] ; sign and exponent of y (denominator)

and     eax, 07fffh              ; clear sy

add     eax, 63                  ; evaluate ey + 63

mov     ebx, [MAIN_NUMER+8+esp]  ; sign and exponent of x (numerator)

and     ebx, 07fffh              ; clear sx

sub     ebx, eax                 ; evaluate the exponent difference (ex - ey)

ja      rem_large               ; if ex > ey + 63, case of large arguments

rem_patch_loop:

mov     eax, [MAIN_DENOM+8+esp]  ; sign and exponent of y (denominator)

and     eax, 07fffh             ; clear sy

add     eax, 10                 ; evaluate ey + 10

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

and     ebx, 07fffh             ; clear sx

sub     ebx, eax                ; evaluate the exponent difference (ex - ey)

js      remainder_hardware_ok   ; safe if ey + 10 > ex

fld     tbyte ptr [MAIN_NUMER+esp]   ; load the numerator

mov     eax, [MAIN_DENOM+8+esp] ; sign and exponent of y (denominator)

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

and     ebx, 07fffh             ; clear sx

mov     ecx, ebx

sub     ebx, eax

and     ebx, 07h

or      ebx, 04h

sub     ecx, ebx

mov     ebx, eax

and     ebx, 08000h             ; keep sy

or      ecx, ebx                ; merge the sign of y

mov     dword ptr [MAIN_DENOM+8+esp], ecx

fld     tbyte ptr [MAIN_DENOM+esp]   ; load the shifted denominator

mov     dword ptr [MAIN_DENOM+8+esp], eax       ; restore the initial denominator

fxch

fprem                           ; this rem is safe

fstp    tbyte ptr [MAIN_NUMER+esp]      ; update the numerator

fstp    st(0)                   ; pop the stack

jmp rem_patch_loop

rem_large:

test    edx, 02h                ; is denominator already saved

jnz     already_saved

fld     tbyte ptr[esp + MAIN_DENOM]

fstp    tbyte ptr[esp + MAIN_DENOM_SAVE]        ; save denominator

already_saved:

; Save user's precision control and institute 80.  The fp ops in

; rem_large_loop must not round to user's precision (if it is less

; than 80) because the hardware would not have done so.  We are

; aping the hardware here, which is all extended.

fnstcw  [esp+MAIN_PREV_CW]      ; save caller's control word

mov     eax, dword ptr[esp + MAIN_PREV_CW]

or      eax, 033fh              ; mask exceptions, pc=80

mov     [esp + MAIN_PATCH_CW], eax

fldcw   [esp + MAIN_PATCH_CW]

mov     eax, [MAIN_DENOM+8+esp] ; sign and exponent of y (denominator)

and     eax, 07fffh             ; clear sy

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

and     ebx, 07fffh             ; clear sx

sub     ebx, eax                ; evaluate the exponent difference

and     ebx, 03fh

or      ebx, 020h

add     ebx, 1

mov     ecx, ebx

mov     eax, [MAIN_DENOM+8+esp] ; sign and exponent of y (denominator)

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

and     ebx, 07fffh             ; clear sx

and     eax, 08000h             ; keep sy

or      ebx, eax                ; merge the sign of y

mov     dword ptr[MAIN_DENOM+8+esp], ebx        ; make ey equal to ex (scaled denominator)

fld     tbyte ptr [MAIN_DENOM+esp]   ; load the scaled denominator

fabs

fld     tbyte ptr [MAIN_NUMER+esp]   ; load the numerator

fabs

rem_large_loop:

fcom

fnstsw  ax

and     eax, 00100h

jnz     rem_no_sub

fsub    st, st(1)

rem_no_sub:

fxch

fmul    qword ptr half

fxch

sub     ecx, 1                  ; decrement the loop counter

jnz     rem_large_loop

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

fstp    tbyte ptr[esp + MAIN_NUMER]     ; save result

fstp    st                      ; toss modified denom

fld     tbyte ptr[esp + MAIN_DENOM_SAVE]

fld     tbyte ptr[big_number]   ; force C2 to be set

fprem

fstp    st

fld     tbyte ptr[esp + MAIN_NUMER]     ; restore saved result

fldcw   [esp + MAIN_PREV_CW]    ; restore caller's control word

and     ebx, 08000h             ; keep sx

jz      rem_done

fchs

jmp     rem_done

remainder_hardware_ok:

fld     tbyte ptr [MAIN_DENOM+esp]   ; load the denominator

fld     tbyte ptr [MAIN_NUMER+esp]   ; load the numerator

fprem                           ; and finally do a remainder

; prem_main_routine end

rem_done:

test    edx, 03h

jz      rem_exit

fnstsw  [esp + MAIN_FPREM_SW]   ; save Q0 Q1 and Q2

test    edx, 01h

jz      do_not_de_scale

; De-scale the result. Mask exception and pc=80 to prevent from fmul

; from user precision (fprem does not round the result).

fnstcw  [esp + MAIN_PREV_CW]    ; save callers control word

mov     eax, [esp + MAIN_PREV_CW]

or      eax, 033fh              ; pc = 80, mask exceptions

mov     [esp + MAIN_PATCH_CW], eax

fldcw   [esp + MAIN_PATCH_CW]

fmul    qword ptr one_shr_64

fldcw   [esp + MAIN_PREV_CW]    ; restore callers CW

do_not_de_scale:

mov     eax, [esp + MAIN_FPREM_SW]

fxch

fstp    st

fld     tbyte ptr[esp + MAIN_DENOM_SAVE]

fxch

and     eax, 04300h             ; restore saved Q0, Q1, Q2

sub     esp, ENV_SIZE

fnstenv [esp]

and     [esp].STATUS_WORD, 0bcffh

or      [esp].STATUS_WORD, eax

fldenv  [esp]

add     esp, ENV_SIZE

rem_exit:

pop     ecx

pop     ebx

pop     eax

CHECKSW                         ; debug only: save status

ret

fprem_common    ENDP

;

; float frem_chk (float numer, float denom)

;

public  frem_chk

frem_chk        PROC    NEAR

push    edx

sub     esp, STACK_SIZE

fld     dword ptr [STACK_SIZE+8+esp]

fstp    tbyte ptr [NUMER+esp]

fld     dword ptr [STACK_SIZE+12+esp]

fstp    tbyte ptr [DENOM+esp]

mov     edx, 0                  ; dx = 1 if denormal extended divisor

call    fprem_common

fxch

fstp    st

add     esp, STACK_SIZE

pop     edx

ret

frem_chk        ENDP

; end frem_chk

;

; double drem_chk (double numer, double denom)

;

public  drem_chk

drem_chk        PROC    NEAR

push    edx

sub     esp, STACK_SIZE

fld     qword ptr [STACK_SIZE+8+esp]

fstp    tbyte ptr [NUMER+esp]

fld     qword ptr [STACK_SIZE+16+esp]

fstp    tbyte ptr [DENOM+esp]

mov     edx, 0                  ; dx = 1 if denormal extended divisor

call    fprem_common

fxch

fstp    st

add     esp, STACK_SIZE

pop     edx

ret

drem_chk        ENDP

; end drem_chk

;

; long double lrem_chk(long double number,long double denom)

;

public  lrem_chk

lrem_chk        PROC    NEAR

fld     tbyte ptr [20+esp]

fld     tbyte ptr [4+esp]

call    fprem_chk

fxch

fstp    st

ret

lrem_chk        ENDP

;

; FPREM: ST = remainder(ST, ST(1))

;

; Compiler version of the FPREM must preserve the arguments in the floating

; point stack.

public  fprem_chk

fprem_chk       PROC    NEAR

push    edx

sub     esp, STACK_SIZE

fstp    tbyte ptr [NUMER+esp]

fstp    tbyte ptr [DENOM+esp]

xor     edx, edx

; prem_main_routine begin

mov     eax,[DENOM+6+esp]       ; exponent and high 16 bits of mantissa

test    eax,07fff0000h          ; check for denormal

jz      denormal

call    fprem_common

add     esp, STACK_SIZE

pop     edx

ret

denormal:

fld     tbyte ptr [DENOM+esp]   ; load the denominator

fld     tbyte ptr [NUMER+esp]   ; load the numerator

mov     eax, [DENOM+esp]        ; test for whole mantissa == 0

or      eax, [DENOM+4+esp]      ; test for whole mantissa == 0

jz      remainder_hardware_ok_l ; denominator is zero

fxch

fstp    tbyte ptr[esp + DENOM_SAVE]     ; save org denominator

fld     tbyte ptr[esp + DENOM]

fxch

or      edx, 02h

;

;       This assures correctly rounded result if pc=64 as well

;

fnstcw  [PREV_CW+esp]           ; save caller's control word

mov     eax, [PREV_CW+esp]

or      eax, 0033fh             ; mask exceptions, pc=80

mov     [PATCH_CW+esp], eax

fldcw   [PATCH_CW+esp]          ; mask exceptions & pc=80

; The denominator is a denormal.  For most numerators, scale both numerator

; and denominator to get rid of denormals.  Then execute the common code

; with the flag set to indicate that the result must be de-scaled.

; For large numerators this won't work because the scaling would cause

; overflow.  In this case we know the numerator is large, the denominator

; is small (denormal), so the exponent difference is also large.  This means

; the rem_large code will be used and this code depends on the difference

; in exponents modulo 64.  Adding 64 to the denominators exponent

; doesn't change the modulo 64 difference.  So we can scale the denominator

; by 64, making it not denormal, and this won't effect the result.

;

; To start with, figure out if numerator is large

mov     eax, [esp + NUMER + 8]  ; load numerator exponent

and     eax, 7fffh              ; isolate numerator exponent

cmp     eax, 7fbeh              ; compare Nexp to Maxexp-64

ja      big_numer_rem_de        ; jif big numerator

; So the numerator is not large scale both numerator and denominator

or      edx, 1                  ; edx = 1, if denormal extended divisor

fmul    qword ptr one_shl_64    ; make numerator not denormal

fstp    tbyte ptr[esp + NUMER]

fmul    qword ptr one_shl_64    ; make denominator not denormal

fstp    tbyte ptr[esp + DENOM]

jmp     scaling_done

; The numerator is large.  Scale only the denominator, which will not

; change the result which we know will be partial.  Set the scale flag

; to false.

big_numer_rem_de:

fstp    st                      ; Toss numerator

fmul    qword ptr one_shl_64    ; make denominator not denormal

fstp    tbyte ptr[esp + DENOM]

; Restore the control word which was fiddled to scale at 80-bit precision.

; Then call the common code.

scaling_done:

fldcw   [esp + PREV_CW]         ; restore callers control word

call    fprem_common

add     esp, STACK_SIZE

pop     edx

ret

remainder_hardware_ok_l:

fprem                           ; and finally do a remainder

CHECKSW

add     esp, STACK_SIZE

pop     edx

ret

fprem_chk       ENDP

; end fprem_chk

;

; FPREM1 code begins here

;

fprem1_common   PROC    NEAR

push    eax

push    ebx

push    ecx

mov     eax, [MAIN_DENOM+6+esp] ; exponent and high 16 bits of mantissa

xor     eax, ONESMASK           ; invert bits that have to be one

test    eax, ONESMASK           ; check bits that have to be one

jnz     remainder1_hardware_ok

shr     eax, 11

and     eax, 0fh

cmp     byte ptr fprem_risc_table[eax], 0     ; check for (1,4,7,a,d)

jz      remainder1_hardware_ok

; The denominator has the bit pattern. Weed out the funny cases like NaNs

; before applying the software version. Our caller guarantees that the

; denominator is not a denormal. Here we check for:

;       denominator     inf, NaN, unnormal

;       numerator       inf, NaN, unnormal, denormal

mov     eax, [MAIN_DENOM+6+esp] ; exponent and high 16 bits of mantissa

and     eax, 07fff0000h         ; mask the exponent only

cmp     eax, 07fff0000h         ; check for INF or NaN

je      remainder1_hardware_ok

mov     eax, [MAIN_NUMER+6+esp] ; exponent and high 16 bits of mantissa

and     eax, 07fff0000h         ; mask the exponent only

jz      remainder1_hardware_ok  ; jif numerator denormal

cmp     eax, 07fff0000h         ; check for INF or NaN

je      remainder1_hardware_ok

mov     eax, [esp + MAIN_NUMER + 4]     ; high mantissa bits - numerator

add     eax, eax                ; set carry if explicit bit set

jnz     remainder1_hardware_ok  ; jmp if numerator is unnormal

mov     eax, [esp + MAIN_DENOM + 4] ; high mantissa bits - denominator

add     eax, eax                ; set carry if explicit bit set

jnz     remainder1_hardware_ok  ; jmp if denominator is unnormal

rem1_patch:

mov     eax, [MAIN_DENOM+8+esp] ; sign and exponent of y (denominator)

and     eax, 07fffh              ; clear sy

add     eax, 63                  ; evaluate ey + 63

mov     ebx, [MAIN_NUMER+8+esp]  ; sign and exponent of x (numerator)

and     ebx, 07fffh              ; clear sx

sub     ebx, eax                 ; evaluate the exponent difference (ex - ey)

ja      rem1_large              ; if ex > ey + 63, case of large arguments

rem1_patch_loop:

mov     eax, [MAIN_DENOM+8+esp]  ; sign and exponent of y (denominator)

and     eax, 07fffh             ; clear sy

add     eax, 10                 ; evaluate ey + 10

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

and     ebx, 07fffh             ; clear sx

sub     ebx, eax                ; evaluate the exponent difference (ex - ey)

js      remainder1_hardware_ok  ; safe if ey + 10 > ex

fld     tbyte ptr [MAIN_NUMER+esp]   ; load the numerator

mov     eax, [MAIN_DENOM+8+esp] ; sign and exponent of y (denominator)

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

and     ebx, 07fffh             ; clear sx

mov     ecx, ebx

sub     ebx, eax

and     ebx, 07h

or      ebx, 04h

sub     ecx, ebx

mov     ebx, eax

and     ebx, 08000h             ; keep sy

or      ecx, ebx                ; merge the sign of y

mov     dword ptr [MAIN_DENOM+8+esp], ecx

fld     tbyte ptr [MAIN_DENOM+esp]   ; load the shifted denominator

mov     dword ptr [MAIN_DENOM+8+esp], eax       ; restore the initial denominator

fxch

fprem                           ; this rem is safe

fstp    tbyte ptr [MAIN_NUMER+esp]      ; update the numerator

fstp    st(0)                   ; pop the stack

jmp rem1_patch_loop

rem1_large:

test    ebx, 02h                ; is denominator already saved

jnz     already_saved1

fld     tbyte ptr[esp + MAIN_DENOM]

fstp    tbyte ptr[esp + MAIN_DENOM_SAVE]        ; save denominator

already_saved1:

; Save user's precision control and institute 80.  The fp ops in

; rem1_large_loop must not round to user's precision (if it is less

; than 80) because the hardware would not have done so.  We are

; aping the hardware here, which is all extended.

fnstcw  [esp+MAIN_PREV_CW]      ; save caller's control word

mov     eax, dword ptr[esp + MAIN_PREV_CW]

or      eax, 033fh              ; mask exceptions, pc=80

mov     [esp + MAIN_PATCH_CW], eax

fldcw   [esp + MAIN_PATCH_CW]

mov     eax, [MAIN_DENOM+8+esp] ; sign and exponent of y (denominator)

and     eax, 07fffh             ; clear sy

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

and     ebx, 07fffh             ; clear sx

sub     ebx, eax                ; evaluate the exponent difference

and     ebx, 03fh

or      ebx, 020h

add     ebx, 1

mov     ecx, ebx

mov     eax, [MAIN_DENOM+8+esp] ; sign and exponent of y (denominator)

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

and     ebx, 07fffh             ; clear sx

and     eax, 08000h             ; keep sy

or      ebx, eax                ; merge the sign of y

mov     dword ptr[MAIN_DENOM+8+esp], ebx        ; make ey equal to ex (scaled denominator)

fld     tbyte ptr [MAIN_DENOM+esp]   ; load the scaled denominator

fabs

fld     tbyte ptr [MAIN_NUMER+esp]   ; load the numerator

fabs

rem1_large_loop:

fcom

fnstsw  ax

and     eax, 00100h

jnz     rem1_no_sub

fsub    st, st(1)

rem1_no_sub:

fxch

fmul    qword ptr half

fxch

sub     ecx, 1                  ; decrement the loop counter

jnz     rem1_large_loop

mov     ebx, [MAIN_NUMER+8+esp] ; sign and exponent of x (numerator)

fstp    tbyte ptr[esp + MAIN_NUMER]     ; save result

fstp    st                      ; toss modified denom

fld     tbyte ptr[esp + MAIN_DENOM_SAVE]

fld     tbyte ptr[big_number]   ; force C2 to be set

fprem1

fstp    st

fld     tbyte ptr[esp + MAIN_NUMER]     ; restore saved result

fldcw   [esp + MAIN_PREV_CW]    ; restore caller's control word

and     ebx, 08000h             ; keep sx

jz      rem1_done

fchs

jmp     rem1_done

remainder1_hardware_ok:

fld     tbyte ptr [MAIN_DENOM+esp]   ; load the denominator

fld     tbyte ptr [MAIN_NUMER+esp]   ; load the numerator

fprem1                           ; and finally do a remainder

; prem1_main_routine end

rem1_done:

test    edx, 03h

jz      rem1_exit

fnstsw  [esp + MAIN_FPREM_SW]   ; save Q0 Q1 and Q2

test    edx, 01h

jz      do_not_de_scale1

; De-scale the result. Mask exception and pc=80 to prevent from fmul

; from user precision (fprem does not round the result).

fnstcw  [esp + MAIN_PREV_CW]    ; save callers control word

mov     eax, [esp + MAIN_PREV_CW]

or      eax, 033fh              ; pc = 80, mask exceptions

mov     [esp + MAIN_PATCH_CW], eax

fldcw   [esp + MAIN_PATCH_CW]

fmul    qword ptr one_shr_64

fldcw   [esp + MAIN_PREV_CW]    ; restore callers CW

do_not_de_scale1:

mov     eax, [esp + MAIN_FPREM_SW]

fxch

fstp    st

fld     tbyte ptr[esp + MAIN_DENOM_SAVE]

fxch

and     eax, 04300h             ; restore saved Q0, Q1, Q2

sub     esp, ENV_SIZE

fnstenv [esp]

and     [esp].STATUS_WORD, 0bcffh

or      [esp].STATUS_WORD, eax

fldenv  [esp]

add     esp, ENV_SIZE

rem1_exit:

pop     ecx

pop     ebx

pop     eax

CHECKSW                         ; debug only: save status

ret

fprem1_common   ENDP

;

; float frem1_chk (float numer, float denom)

;

public  frem1_chk

frem1_chk       PROC    NEAR

push    edx

sub     esp, STACK_SIZE

fld     dword ptr [STACK_SIZE+8+esp]

fstp    tbyte ptr [NUMER+esp]

fld     dword ptr [STACK_SIZE+12+esp]

fstp    tbyte ptr [DENOM+esp]

mov     edx, 0                  ; dx = 1 if denormal extended divisor

call    fprem1_common

fxch

fstp    st

add     esp, STACK_SIZE

pop     edx

ret

frem1_chk       ENDP

; end frem1_chk

;

; double drem1_chk (double numer, double denom)

;

public  drem1_chk

drem1_chk       PROC    NEAR

push    edx

sub     esp, STACK_SIZE

fld     qword ptr [STACK_SIZE+8+esp]

fstp    tbyte ptr [NUMER+esp]

fld     qword ptr [STACK_SIZE+16+esp]

fstp    tbyte ptr [DENOM+esp]

mov     edx, 0                  ; dx = 1 if denormal extended divisor

call    fprem1_common

fxch

fstp    st

add     esp, STACK_SIZE

pop     edx

ret

drem1_chk       ENDP

; end drem1_chk

;

; long double lrem1_chk(long double number,long double denom)

;

public  lrem1_chk

lrem1_chk       PROC    NEAR

fld     tbyte ptr [20+esp]

fld     tbyte ptr [4+esp]

call    fprem1_chk

fxch

fstp    st

ret

lrem1_chk       ENDP

;

; FPREM1: ST = remainder(ST, ST(1)) - IEEE version of rounding

;

; Compiler version of the FPREM must preserve the arguments in the floating

; point stack.

public  fprem1_chk

fprem1_chk      PROC    NEAR

push    edx

sub     esp, STACK_SIZE

fstp    tbyte ptr [NUMER+esp]

fstp    tbyte ptr [DENOM+esp]

mov     edx, 0

; prem1_main_routine begin

mov     eax,[DENOM+6+esp]       ; exponent and high 16 bits of mantissa

test    eax,07fff0000h          ; check for denormal

jz      denormal1

call    fprem1_common

add     esp, STACK_SIZE

pop     edx

ret

denormal1:

fld     tbyte ptr [DENOM+esp]   ; load the denominator

fld     tbyte ptr [NUMER+esp]   ; load the numerator

mov     eax, [DENOM+esp]        ; test for whole mantissa == 0

or      eax, [DENOM+4+esp]      ; test for whole mantissa == 0

jz      remainder1_hardware_ok_l ; denominator is zero

fxch

fstp    tbyte ptr[esp + DENOM_SAVE]     ; save org denominator

fld     tbyte ptr[esp + DENOM]

fxch

or      edx, 02h

;

;       This assures correctly rounded result if pc=64 as well

;

fnstcw  [PREV_CW+esp]           ; save caller's control word

mov     eax, [PREV_CW+esp]

or      eax, 0033fh             ; mask exceptions, pc=80

mov     [PATCH_CW+esp], eax

fldcw   [PATCH_CW+esp]          ; mask exceptions & pc=80

; The denominator is a denormal.  For most numerators, scale both numerator

; and denominator to get rid of denormals.  Then execute the common code

; with the flag set to indicate that the result must be de-scaled.

; For large numerators this won't work because the scaling would cause

; overflow.  In this case we know the numerator is large, the denominator

; is small (denormal), so the exponent difference is also large.  This means

; the rem1_large code will be used and this code depends on the difference

; in exponents modulo 64.  Adding 64 to the denominators exponent

; doesn't change the modulo 64 difference.  So we can scale the denominator

; by 64, making it not denormal, and this won't effect the result.

;

; To start with, figure out if numerator is large

mov     eax, [esp + NUMER + 8]  ; load numerator exponent

and     eax, 7fffh              ; isolate numerator exponent

cmp     eax, 7fbeh              ; compare Nexp to Maxexp-64

ja      big_numer_rem1_de       ; jif big numerator

; So the numerator is not large scale both numerator and denominator

or      edx, 1                  ; edx = 1, if denormal extended divisor

fmul    qword ptr one_shl_64    ; make numerator not denormal

fstp    tbyte ptr[esp + NUMER]

fmul    qword ptr one_shl_64    ; make denominator not denormal

fstp    tbyte ptr[esp + DENOM]

jmp     scaling_done1

; The numerator is large.  Scale only the denominator, which will not

; change the result which we know will be partial.  Set the scale flag

; to false.

big_numer_rem1_de:

fstp    st                      ; Toss numerator

fmul    qword ptr one_shl_64    ; make denominator not denormal

fstp    tbyte ptr[esp + DENOM]

; Restore the control word which was fiddled to scale at 80-bit precision.

; Then call the common code.

scaling_done1:

fldcw   [esp + PREV_CW]         ; restore callers control word

call    fprem1_common

add     esp, STACK_SIZE

pop     edx

ret

remainder1_hardware_ok_l:

fprem                           ; and finally do a remainder

CHECKSW

add     esp, STACK_SIZE

pop     edx

ret

fprem1_chk      ENDP

; end fprem1_chk

ifdef   DEBUG

public  fpinit

fpinit  PROC    NEAR

fninit

ret

fpinit  ENDP

endif

CODE32 ENDS

END

fptan32.asm:

------------

; static char sccs_id[] = "@(#)fptan32.asm      1.1 12/22/94 12:49:13";

;

; This code is being published by Intel to users of the Pentium(tm)

; processor.  Recipients are authorized to copy, modify, compile, use and

; distribute the code.

;

; Intel makes no warranty of any kind with regard to this code, including

; but not limited to, implied warranties or merchantability and fitness for

; a particular purpose. Intel assumes no responsibility for any errors that

; may appear in this code.

;

; No patent licenses are granted, express or implied.

;

;

;  The following code is a PRELIMINARY IMPLEMENTATION of a

;  software patch for the floating point divide instructions.

;

;

.386

.387

PATCH_CW        EQU     00ch

PREV_CW         EQU     010h

COSINE          EQU     0               ; These two are overlaid because they

ANGLE           EQU     0               ; are not live at the same time.

STACK_SIZE      EQU     014h

ONESMASK        EQU     0e000000h

DATA32  SEGMENT DWORD USE32 PUBLIC 'DATA'

fdiv_risk_table DB      0, 1, 0, 0, 4, 0, 0, 7, 0, 0, 10, 0, 0, 13, 0, 0

fdiv_scale      DD      03f700000h              ; 0.9375

one_shl_63      DD      05f000000h

DATA32  ENDS

CODE32  SEGMENT   DWORD USE32 PUBLIC 'CODE'

assume cs:CODE32, ds:FLAT, es:FLAT, ss:nothing

public  fptan_chk

public  _fptan_chk

;

;  PRELIMINARY VERSION for register-register divides.

;

fptan_chk PROC  NEAR

_fptan_chk:

push    eax

sub     esp, STACK_SIZE

fstp    tbyte ptr [esp+ANGLE]

mov     eax, [esp+ANGLE+8]

and     eax, 07fffh

jz      use_hardware            ; denormals, ...

cmp     eax, 07fffh

je      use_hardware            ; NaNs, infinities, ...

mov     eax, [esp+ANGLE+4]

add     eax, eax

jnc     use_hardware            ; unnormals (explicit 1 missing)

fld     tbyte ptr [esp+ANGLE]

;

; Check for proper parameter range ( |<angle>| < 2^63)

;

fabs

fcomp   one_shl_63

fnstsw  ax

sahf

jae     use_hardware

fld     tbyte ptr [esp+ANGLE]

fsincos

fstp    tbyte ptr [esp+COSINE]

fld     tbyte ptr [esp+COSINE]  ; load the denominator (cos(x))

mov     eax, [esp+COSINE+4]     ; get mantissa bits 32-64

add     eax,eax                 ; shift the one's bit onto carry

xor     eax, ONESMASK           ; invert the bits that must be ones

test    eax, ONESMASK           ; and make sure they are all ones

jz      scale_if_needed         ; if all are one scale numbers

fdivp   st(1), st               ; use of hardware is OK.

fld1                            ; push 1.0 onto FPU stack

add     esp, STACK_SIZE

pop     eax

ret

scale_if_needed:

shr     eax, 28                 ; keep first 4 bits after point

cmp     fdiv_risk_table[eax], ah        ; check for (1,4,7,a,d)

jnz     divide_scaled           ; are in potential problem area

fdivp   st(1), st               ; use of hardware is OK.

fld1                            ; push 1.0 onto FPU stack

add     esp, STACK_SIZE

pop     eax

ret

divide_scaled:

fwait                           ; catch preceding exceptions

fstcw   [esp+PREV_CW]           ; save caller's control word

mov     eax, [esp+PREV_CW]

or      eax, 033fh              ; mask exceptions, pc=80

mov     [esp+PATCH_CW], eax

fldcw   [esp+PATCH_CW]          ; mask exceptions & pc=80

fmul    fdiv_scale              ; scale denominator by 15/16

fxch

fmul    fdiv_scale              ; scale numerator by 15/16

fxch

; This assures correctly rounded result if pc=64 as well

fldcw   [esp+PREV_CW]           ; restore caller's control word

fdivp   st(1), st               ; use of hardware is OK.

fld1                            ; push 1.0 onto FPU stack

add     esp, STACK_SIZE

pop     eax

ret

use_hardware:

fld     tbyte ptr [esp+ANGLE]

fptan

add     esp, STACK_SIZE

pop     eax

ret

fptan_chk       ENDP

CODE32  ENDS

end

patch32.asm:

------------

; static char sccs_id[] = "@(#)patch32.asm      1.2  1/12/95  15:39:55";

;

; This code is being published by Intel to users of the Pentium(tm)

; processor.  Recipients are authorized to copy, modify, compile, use and

; distribute the code.

;

; Intel makes no warranty of any kind with regard to this code, including

; but not limited to, implied warranties or merchantability and fitness for

; a particular purpose. Intel assumes no responsibility for any errors that

; may appear in this code.

;

; No patent licenses are granted, express or implied.

;

;

.386

.387

DENOM           EQU     0

NUMER           EQU     12

PREV_CW         EQU     28      ; 24 + 4 (return size)

PATCH_CW        EQU     32      ; 28 + 4 (return size)

DENOM_SAVE      EQU     32

MAIN_DENOM      EQU     4

MAIN_NUMER      EQU     16

SPILL_SIZE      EQU     12

MEM_OPERAND     EQU     8

STACK_SIZE      EQU     44

SPILL_MEM_OPERAND       EQU     20

ONESMASK        EQU     0e000000h

SINGLE_NAN      EQU     07f800000h

DOUBLE_NAN      EQU     07ff00000h

C023MASK        EQU     04500h

EMPTY_MASK      EQU     04100h

ILLEGAL_OPC     EQU     6

fdivr_st        MACRO   reg_index, reg_index_minus1

fstp    tbyte ptr [esp+DENOM]

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fstp    tbyte ptr [esp+NUMER]

call    fdiv_main_routine

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fld     tbyte ptr [esp+NUMER]

fxch    st(reg_index)

add     esp, STACK_SIZE

ENDM

fdivr_sti       MACRO   reg_index, reg_index_minus1

fstp    tbyte ptr [esp+NUMER]

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fstp    tbyte ptr [esp+DENOM]

call    fdiv_main_routine

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fld     tbyte ptr [esp+NUMER]

add     esp, STACK_SIZE

ENDM

fdivrp_sti      MACRO   reg_index, reg_index_minus1

fstp    tbyte ptr [esp+NUMER]

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fstp    tbyte ptr [esp+DENOM]

call    fdiv_main_routine

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

add     esp, STACK_SIZE

ENDM

fdiv_st         MACRO   reg_index, reg_index_minus1

fstp    tbyte ptr [esp+NUMER]

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fld     st

fstp    tbyte ptr [esp+DENOM]

fstp    tbyte ptr [esp+DENOM_SAVE]      ; save original denom,

call    fdiv_main_routine

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fld     tbyte ptr [esp+DENOM_SAVE]

fxch    st(reg_index)

add     esp, STACK_SIZE

ENDM

fdiv_sti        MACRO   reg_index, reg_index_minus1

fxch    st(reg_index)

fstp    tbyte ptr [esp+NUMER]

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fld     st

fstp    tbyte ptr [esp+DENOM]

fstp    tbyte ptr [esp+DENOM_SAVE]      ; save original denom,

call    fdiv_main_routine

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fld     tbyte ptr [esp+DENOM_SAVE]

add     esp, STACK_SIZE

ENDM

fdivp_sti       MACRO   reg_index, reg_index_minus1

fstp    tbyte ptr [esp+DENOM]

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

fstp    tbyte ptr [esp+NUMER]

call    fdiv_main_routine

IF      reg_index_minus1 GE 1

fxch    st(reg_index_minus1)

ENDIF

add     esp, STACK_SIZE

ENDM

CODE32  SEGMENT DWORD USE32 PUBLIC 'CODE'

CODE32  ENDS

DATA32  SEGMENT DWORD USE32 PUBLIC 'DATA'

DATA32  ENDS

CONST32 SEGMENT DWORD USE32 PUBLIC 'CONST'

CONST32 ENDS

BSS32   SEGMENT DWORD USE32 PUBLIC 'BSS'

BSS32   ENDS

DGROUP  GROUP CONST32, BSS32, DATA32

DATA32  SEGMENT DWORD USE32 PUBLIC 'DATA'

fdiv_risc_table DB      0, 1, 0, 0, 4, 0, 0, 7, 0, 0, 10, 0, 0, 13, 0, 0

fdiv_scale_1    DD      03f700000h              ;0.9375

fdiv_scale_2    DD      03f880000h              ;1.0625

one_shl_63      DD      05f000000h

;

;  This is the flag that is tested by all the compiler branches

;  before the divide instruction.

;

;       0 = unknown whether running on a part with the flaw

;       1 = running on a part with no flaw

;       -1= running on a part with the flaw

;

EXTRN   _fdiv_chk_flag:DWORD

;

;  The following are initializers for known values that cause

;  the divide problem.

;

dispatch_table DD       offset FLAT:label0

DD      offset FLAT:label1

DD      offset FLAT:label2

DD      offset FLAT:label3

DD      offset FLAT:label4

DD      offset FLAT:label5

DD      offset FLAT:label6

DD      offset FLAT:label7

DD      offset FLAT:label8

DD      offset FLAT:label9

DD      offset FLAT:label10

DD      offset FLAT:label11

DD      offset FLAT:label12

DD      offset FLAT:label13

DD      offset FLAT:label14

DD      offset FLAT:label15

DD      offset FLAT:label16

DD      offset FLAT:label17

DD      offset FLAT:label18

DD      offset FLAT:label19

DD      offset FLAT:label20

DD      offset FLAT:label21

DD      offset FLAT:label22

DD      offset FLAT:label23

DD      offset FLAT:label24

DD      offset FLAT:label25

DD      offset FLAT:label26

DD      offset FLAT:label27

DD      offset FLAT:label28

DD      offset FLAT:label29

DD      offset FLAT:label30

DD      offset FLAT:label31

DD      offset FLAT:label32

DD      offset FLAT:label33

DD      offset FLAT:label34

DD      offset FLAT:label35

DD      offset FLAT:label36

DD      offset FLAT:label37

DD      offset FLAT:label38

DD      offset FLAT:label39

DD      offset FLAT:label40

DD      offset FLAT:label41

DD      offset FLAT:label42

DD      offset FLAT:label43

DD      offset FLAT:label44

DD      offset FLAT:label45

DD      offset FLAT:label46

DD      offset FLAT:label47

DD      offset FLAT:label48

DD      offset FLAT:label49

DD      offset FLAT:label50

DD      offset FLAT:label51

DD      offset FLAT:label52

DD      offset FLAT:label53

DD      offset FLAT:label54

DD      offset FLAT:label55

DD      offset FLAT:label56

DD      offset FLAT:label57

DD      offset FLAT:label58

DD      offset FLAT:label59

DD      offset FLAT:label60

DD      offset FLAT:label61

DD      offset FLAT:label62

DD      offset FLAT:label63

FPU_STATE       STRUC

CONTROL_WORD    DW      ?

reserved_1      DW      ?

STATUS_WORD     DD      ?

TAG_WORD        DD      ?

IP_OFFSET       DD      ?

CS_SLCT         DW      ?

OPCODE          DW      ?

DATA_OFFSET     DD      ?

OPERAND_SLCT    DW      ?

reserved_4      DW      ?

FPU_STATE       ENDS

ENV_SIZE        EQU     28

DATA32  ENDS

CODE32  SEGMENT   DWORD USE32 PUBLIC 'CODE'

assume cs:CODE32, ds:FLAT, es:FLAT, ss:nothing

EXTRN   _FDIV_DETECT:NEAR

;

;  PRELIMINARY VERSION for register-register divides.

;

; In this implementation the

; fdiv_main_routine is called,

; therefore all the stack frame

; locations are adjusted for the

; return pointer.

fdiv_main_routine PROC  NEAR

fld     tbyte ptr [esp+MAIN_NUMER]      ; load the numerator

fld     tbyte ptr [esp+MAIN_DENOM]      ; load the denominator

retry:

;  The following three lines test for denormals and zeros.

;  A denormal or zero has a 0 in the explicit digit to the left of the

;  binary point.  Since that bit is the high bit of the word, adding

;  it to itself will produce a carry if and only if the number is not

;  denormal or zero.

;

mov     eax, [esp+MAIN_DENOM+4] ; get mantissa bits 32-64

add     eax,eax                 ; shift the one's bit onto carry

jnc     denormal                ; if no carry, we're denormal

;  The following three lines test the three bits after the four bit

;  pattern (1,4,7,a,d).  If these three bits are not all one, then

;  the denominator cannot expose the flaw.  This condition is tested by

;  inverting the bits and testing that all are equal to zero afterward.

xor     eax, ONESMASK           ; invert the bits that must be ones

test    eax, ONESMASK           ; and make sure they are all ones

jz      scale_if_needed         ; if all are one scale numbers

fdivp   st(1), st               ; use of hardware is OK.

ret

;

;  Now we test the four bits for one of the five patterns.

;

scale_if_needed:

shr     eax, 28                 ; keep first 4 bits after point

cmp     byte ptr fdiv_risc_table[eax], 0        ; check for (1,4,7,a,d)

jnz     divide_scaled           ; are in potential problem area

fdivp   st(1), st               ; use of hardware is OK.

ret

divide_scaled:

mov     eax, [esp + MAIN_DENOM+8]       ; test denominator exponent

and     eax, 07fffh             ; if pseudodenormal ensure that only

jz      invalid_denom           ; invalid exception flag is set

cmp     eax, 07fffh             ; if NaN or infinity  ensure that only

je      invalid_denom           ; invalid exception flag is set

;

;  The following six lines turn off exceptions and set the

;  precision control to 80 bits.  The former is necessary to

;  force any traps to be taken at the divide instead of the scaling

;  code.  The latter is necessary in order to get full precision for

;  codes with incoming 32 and 64 bit precision settings.  If

;  it can be guaranteed that before reaching this point, the underflow

;  exception is masked and the precision control is at 80 bits, these

;  six lines can be omitted.

;

fnstcw  [esp+PREV_CW]           ; save caller's control word

mov     eax, [esp+PREV_CW]

or      eax, 033fh              ; mask exceptions, pc=80

and     eax, 0f3ffh             ; set rounding mode to nearest

mov     [esp+PATCH_CW], eax

fldcw   [esp+PATCH_CW]          ; mask exceptions & pc=80

;  The following lines check the numerator exponent before scaling.

;  This in order to prevent undeflow when scaling the numerator,

;  which will cause a denormal exception flag to be set when the

;  actual divide is preformed. This flag would not have been set

;  normally. If there is a risk of underflow, the scale factor is

;  17/16 instead of 15/16.

;

mov     eax, [esp+MAIN_NUMER+8] ; test numerator exponent

and     eax, 07fffh

cmp     eax, 00001h

je      small_numer

fmul    fdiv_scale_1            ; scale denominator by 15/16

fxch

fmul    fdiv_scale_1            ; scale numerator by 15/16

fxch

;

;  The next line restores the users control word.  If the incoming

;  control word had the underflow exception masked and precision

;  control set to 80 bits, this line can be omitted.

;

fldcw   [esp+PREV_CW]           ; restore caller's control word

fdivp   st(1), st               ; use of hardware is OK.

ret

small_numer:

fmul    fdiv_scale_2            ; scale denominator by 17/16

fxch

fmul    fdiv_scale_2            ; scale numerator by 17/16

fxch

;

;  The next line restores the users control word.  If the incoming

;  control word had the underflow exception masked and precision

;  control set to 80 bits, this line can be omitted.

;

fldcw   [esp+PREV_CW]           ; restore caller's control word

fdivp   st(1), st               ; use of hardware is OK.

ret

denormal:

mov     eax, [esp+MAIN_DENOM]   ; test for whole mantissa == 0

or      eax, [esp+MAIN_DENOM+4] ; test for whole mantissa == 0

jnz     denormal_divide_scaled  ; denominator is not zero

invalid_denom:                          ; zero or invalid denominator

fdivp   st(1), st               ; use of hardware is OK.

ret

denormal_divide_scaled:

mov     eax, [esp + MAIN_DENOM + 8]     ; get exponent

and     eax, 07fffh             ; check for zero exponent

jnz     invalid_denom           ;

;

;  The following six lines turn off exceptions and set the

;  precision control to 80 bits.  The former is necessary to

;  force any traps to be taken at the divide instead of the scaling

;  code.  The latter is necessary in order to get full precision for

;  codes with incoming 32 and 64 bit precision settings.  If

;  it can be guaranteed that before reaching this point, the underflow

;  exception is masked and the precision control is at 80 bits, these

;  six lines can be omitted.

;

fnstcw  [esp+PREV_CW]           ; save caller's control word

mov     eax, [esp+PREV_CW]

or      eax, 033fh              ; mask exceptions, pc=80

and     eax, 0f3ffh             ; set rounding mode to nearest

mov     [esp+PATCH_CW], eax

fldcw   [esp+PATCH_CW]          ; mask exceptions & pc=80

mov     eax, [esp + MAIN_NUMER +8]      ; test numerator exponent

and     eax, 07fffh             ; check for denormal numerator

je      denormal_numer

cmp     eax, 07fffh             ; NaN or infinity

je      invalid_numer

mov     eax, [esp + MAIN_NUMER + 4]     ; get bits 32..63 of mantissa

add     eax, eax                ; shift the first bit into carry

jnc     invalid_numer           ; if there is no carry, we have an

; invalid numer

jmp     numer_ok

denormal_numer:

mov     eax, [esp + MAIN_NUMER + 4]     ; get bits 32..63 of mantissa

add     eax, eax                ; shift the first bit into carry

jc      invalid_numer           ; if there is a carry, we have an

; invalid numer

numer_ok:

fxch

fstp    st                      ; pop numerator

fld     st                      ; make copy of denominator

fmul    dword ptr[one_shl_63]   ; make denominator not denormal

fstp    tbyte ptr [esp+MAIN_DENOM]      ; save modified denominator

fld     tbyte ptr [esp+MAIN_NUMER]      ; load numerator

fxch                            ; restore proper order

fwait

;  The next line restores the users control word.  If the incoming

;  control word had the underflow exception masked and precision

;  control set to 80 bits, this line can be omitted.

;

fldcw   [esp+PREV_CW]           ; restore caller's control word

jmp     retry                   ; start the whole thing over

invalid_numer:

;

;  The next line restores the users control word.  If the incoming

;  control word had the underflow exception masked and precision

;  control set to 80 bits, this line can be omitted.

;

fldcw   [esp + PREV_CW]

fdivp   st(1), st               ; use of hardware is OK.

ret

fdiv_main_routine       ENDP

public  _fdiv_r

_fdiv_r      PROC    NEAR

cmp     _fdiv_chk_flag, 0

jne     flaw_detected1

push    eax

sub     esp, 12

fstp    tbyte ptr [esp]

call    _FDIV_DETECT

fld     tbyte ptr [esp]

add     esp, 12

pop     eax

flaw_detected1:

sub     esp, STACK_SIZE

jmp     dword ptr dispatch_table[eax*4]

label0:

fdiv    st,st(0)                ; D8 F0         FDIV    ST,ST(0)

add     esp, STACK_SIZE

ret

label1:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label2:

fdivr   st,st(0)                ; D8 F8         FDIVR   ST,ST(0)

add     esp, STACK_SIZE

ret

label3:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label4:

fdiv    st(0),st                ; DC F8/D8 F0   FDIV    ST(0),ST

add     esp, STACK_SIZE

ret

label5:

fdivp   st(0),st                ; DE F8         FDIVP   ST(0),ST

add     esp, STACK_SIZE

ret

label6:

fdivr   st(0),st                ; DC F0/DE F0   FDIVR   ST(0),ST

add     esp, STACK_SIZE

ret

label7:

fdivrp  st(0),st                ; DE F0         FDIVRP  ST(0),ST

add     esp, STACK_SIZE

ret

label8:

fdiv_st 1 0

ret

label9:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label10:

fdivr_st 1 0

ret

label11:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label12:

fdiv_sti 1 0

ret

label13:

fdivp_sti 1 0

ret

label14:

fdivr_sti 1 0

ret

label15:

fdivrp_sti 1 0

ret

label16:

fdiv_st 2 1

ret

label17:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label18:

fdivr_st 2 1

ret

label19:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label20:

fdiv_sti 2 1

ret

label21:

fdivp_sti 2 1

ret

label22:

fdivr_sti 2 1

ret

label23:

fdivrp_sti 2 1

ret

label24:

fdiv_st 3 2

ret

label25:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label26:

fdivr_st 3 2

ret

label27:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label28:

fdiv_sti 3 2

ret

label29:

fdivp_sti 3 2

ret

label30:

fdivr_sti 3 2

ret

label31:

fdivrp_sti 3 2

ret

label32:

fdiv_st 4 3

ret

label33:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label34:

fdivr_st 4 3

ret

label35:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label36:

fdiv_sti 4 3

ret

label37:

fdivp_sti 4 3

ret

label38:

fdivr_sti 4 3

ret

label39:

fdivrp_sti 4 3

ret

label40:

fdiv_st 5 4

ret

label41:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label42:

fdivr_st 5 4

ret

label43:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label44:

fdiv_sti 5 4

ret

label45:

fdivp_sti 5 4

ret

label46:

fdivr_sti 5 4

ret

label47:

fdivrp_sti 5 4

ret

label48:

fdiv_st 6 5

ret

label49:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label50:

fdivr_st 6 5

ret

label51:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label52:

fdiv_sti 6 5

ret

label53:

fdivp_sti 6 5

ret

label54:

fdivr_sti 6 5

ret

label55:

fdivrp_sti 6 5

ret

label56:

fdiv_st 7 6

ret

label57:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label58:

fdivr_st 7 6

ret

label59:

add     esp, STACK_SIZE

int     ILLEGAL_OPC

label60:

fdiv_sti 7 6

ret

label61:

fdivp_sti 7 6

ret

label62:

fdivr_sti 7 6

ret

label63:

fdivrp_sti 7 6

ret

_fdiv_r      ENDP

fdivp_sti_st    PROC    NEAR

; for calling from mem routines

cmp     _fdiv_chk_flag, 0

jne     flaw_detected2

sub     esp, 12

fstp    tbyte ptr [esp]

call    _FDIV_DETECT

fld     tbyte ptr [esp]

add     esp, 12

flaw_detected2:

sub     esp, STACK_SIZE

fdivp_sti 1 0

ret

fdivp_sti_st    ENDP

fdivrp_sti_st   PROC    NEAR

; for calling from mem routines

cmp     _fdiv_chk_flag, 0

jne     flaw_detected3

sub     esp, 12

fstp    tbyte ptr [esp]

call    _FDIV_DETECT

fld     tbyte ptr [esp]

add     esp, 12

flaw_detected3:

sub     esp, STACK_SIZE

fdivrp_sti 1 0

ret

fdivrp_sti_st   ENDP

;

;  PRELIMINARY VERSIONS of the routines for register-memory

;  divide instructions

;

;;; FDIV_M32 - FDIV m32real FIX

;;

;;      Input : Value of the m32real in the top of STACK

;;

;;      Output: Result of FDIV in ST

PUBLIC  _fdiv_m32

_fdiv_m32       PROC    NEAR

push    eax                             ; save eax

mov     eax, [esp + MEM_OPERAND]        ; check for

and     eax, SINGLE_NAN                 ; NaN

cmp     eax, SINGLE_NAN                 ;

je      memory_divide_m32               ;

call    fdiv_tos_valid                  ; check if (TOS-1) is empty

cmp     eax, 0                          ; is TOS-1 not empty?

je      spill_fpstack                   ; spill TOS-1 reg

fld     dword ptr[esp + MEM_OPERAND]    ; load m32real in ST

call    fdivp_sti_st                    ; do actual divide

pop     eax

ret

spill_fpstack:

sub     esp, SPILL_SIZE                 ; make temp space

fstp    tbyte ptr[esp ]                 ; save user's ST(1)

fld     dword ptr[esp + SPILL_MEM_OPERAND] ; load m32 real

call    fdivp_sti_st                    ; do actual divide

fld     tbyte ptr[esp]                  ; restore user's (TOS-1)

fincstp                                 ; restore FP stack

add     esp, SPILL_SIZE

pop     eax

ret

memory_divide_m32:

fdiv    dword ptr[esp + MEM_OPERAND]    ; do actual divide

pop     eax

ret

_fdiv_m32       ENDP

;;; FDIV_M64 - FDIV m64real FIX

;;

;;      Input : Value of the m64real in the top of STACK

;;

;;      Output: Result of FDIV in ST

PUBLIC  _fdiv_m64

_fdiv_m64       PROC    NEAR

push    eax                             ; save eax

mov     eax, [esp + MEM_OPERAND + 4]    ; check for

and     eax, DOUBLE_NAN                 ; NaN

cmp     eax, DOUBLE_NAN                 ;

je      memory_divide_m64               ;

call    fdiv_tos_valid                  ; check if (TOS-1) is empty

cmp     eax, 0                          ; is TOS-1 not empty?

je      spill_fpstack_m64               ; spill TOS-1 reg

fld     qword ptr[esp + MEM_OPERAND]    ; load m64real in ST

call    fdivp_sti_st                    ; do actual divide

pop     eax

ret

spill_fpstack_m64:

sub     esp, SPILL_SIZE                 ; make temp space

fstp    tbyte ptr[esp]                  ; save user's ST(1)

fld     qword ptr[esp + SPILL_MEM_OPERAND] ; load m64real

call    fdivp_sti_st                    ; do actual divide

fld     tbyte ptr[esp]                  ; restore user's (TOS-1)

fincstp                                 ; restore FP stack

add     esp, SPILL_SIZE

pop     eax

ret

memory_divide_m64:

fdiv    qword ptr[esp + MEM_OPERAND]    ; do actual divide

pop     eax

ret

_fdiv_m64       ENDP

;;; FDIV_M16I - FDIV m16int FIX

;;

;;      Input : Value of the m16int in the top of STACK

;;

;;      Output: Result of FDIV in ST

PUBLIC  _fdiv_m16i

_fdiv_m16i      PROC    NEAR

push    eax                             ; save eax

call    fdiv_tos_valid                  ; check if (TOS-1) is empty

cmp     eax, 0                          ; is TOS-1 not empty?

je      spill_fpstack_m16i              ; spill TOS-1 reg

fild    word ptr[esp + MEM_OPERAND]     ; load m16int in ST

call    fdivp_sti_st                    ; do actual divide

pop     eax

ret

spill_fpstack_m16i:

sub     esp, SPILL_SIZE                 ; make temp space

fstp    tbyte ptr[esp ]                 ; save user's ST(1)

fild    word ptr[esp + SPILL_MEM_OPERAND] ; load m16int

call    fdivp_sti_st                    ; do actual divide

fld     tbyte ptr[esp]                  ; restore user's (TOS-1)

fincstp                                 ; restore FP stack

add     esp, SPILL_SIZE

pop     eax

ret

_fdiv_m16i      ENDP

;;; FDIV_M32I - FDIV m16int FIX

;;

;;      Input : Value of the m16int in the top of STACK

;;

;;      Output: Result of FDIV in ST

PUBLIC  _fdiv_m32i

_fdiv_m32i      PROC    NEAR

push    eax                             ; save eax

call    fdiv_tos_valid                  ; check if (TOS-1) is empty

cmp     eax, 0                          ; is TOS-1 not empty?

je      spill_fpstack_m32i              ; spill TOS-1 reg

fild    dword ptr[esp + MEM_OPERAND]    ; load m32int in ST

call    fdivp_sti_st                    ; do actual divide

pop     eax

ret

spill_fpstack_m32i:

sub     esp, SPILL_SIZE                 ; make temp space

fstp    tbyte ptr[esp ]                 ; save user's ST(1)

fild    dword ptr[esp + SPILL_MEM_OPERAND] ; load m32int

call    fdivp_sti_st                    ; do actual divide

fld     tbyte ptr[esp]                  ; restore user's (TOS-1)

fincstp                                 ; restore FP stack

add     esp, SPILL_SIZE

pop     eax

ret

_fdiv_m32i      ENDP

;;; FDIVR_M32 - FDIVR m32real FIX

;;

;;      Input : Value of the m32real in the top of STACK

;;

;;      Output: Result of FDIVR in ST

PUBLIC  _fdivr_m32

_fdivr_m32      PROC    NEAR

push    eax                             ; save eax

mov     eax, [esp + MEM_OPERAND]        ; check for

and     eax, SINGLE_NAN                 ; NaN

cmp     eax, SINGLE_NAN                 ;

je      memory_divide_m32r              ;

call    fdiv_tos_valid                  ; check if (TOS-1) is empty

cmp     eax, 0                          ; is TOS-1 not empty?

je      spill_fpstack_m32r              ; spill TOS-1 reg

fld     dword ptr[esp + MEM_OPERAND]    ; load m32real in ST

call    fdivrp_sti_st                   ; do actual divide

pop     eax

ret

spill_fpstack_m32r:

sub     esp, SPILL_SIZE                 ; make temp space

fstp    tbyte ptr[esp ]                 ; save user's ST(1)

fld     dword ptr[esp + SPILL_MEM_OPERAND] ; load m32 real

call    fdivrp_sti_st                   ; do actual divide

fld     tbyte ptr[esp]                  ; restore user's (TOS-1)

fincstp                                 ; restore FP stack

add     esp, SPILL_SIZE

pop     eax

ret

memory_divide_m32r:

fdivr   dword ptr[esp + MEM_OPERAND]    ; do actual divide

pop     eax

ret

_fdivr_m32      ENDP

;;; FDIVR_M64 - FDIVR m64real FIX

;;

;;      Input : Value of the m64real in the top of STACK

;;

;;      Output: Result of FDIVR in ST

PUBLIC  _fdivr_m64

_fdivr_m64      PROC    NEAR

push    eax                             ; save eax

mov     eax, [esp + MEM_OPERAND + 4]    ; check for

and     eax, DOUBLE_NAN                 ; NaN

cmp     eax, DOUBLE_NAN                 ;

je      memory_divide_m64r              ;

call    fdiv_tos_valid                  ; check if (TOS-1) is empty

cmp     eax, 0                          ; is TOS-1 not empty?

je      spill_fpstack_m64r              ; spill TOS-1 reg

fld     qword ptr[esp + MEM_OPERAND]    ; load m64real in ST

call    fdivrp_sti_st                   ; do actual divide

pop     eax

ret

spill_fpstack_m64r:

sub     esp, SPILL_SIZE                 ; make temp space

fstp    tbyte ptr[esp ]                 ; save user's ST(1)

fld     qword ptr[esp + SPILL_MEM_OPERAND] ; load m64real

call    fdivrp_sti_st                   ; do actual divide

fld     tbyte ptr[esp]                  ; restore user's (TOS-1)

fincstp                                 ; restore FP stack

add     esp, SPILL_SIZE

pop     eax

ret

memory_divide_m64r:

fdivr   qword ptr[esp + MEM_OPERAND]    ; do actual divide

pop     eax

ret

_fdivr_m64      ENDP

;;; FDIVR_M16I - FDIVR m16int FIX

;;

;;      Input : Value of the m16int in the top of STACK

;;

;;      Output: Result of FDIVR in ST

PUBLIC  _fdivr_m16i

_fdivr_m16i     PROC    NEAR

push    eax                             ; save eax

call    fdiv_tos_valid                  ; check if (TOS-1) is empty

cmp     eax, 0                          ; is TOS-1 not empty?

je      spill_fpstack_m16ir             ; spill TOS-1 reg

fild    word ptr[esp + MEM_OPERAND]     ; load m16int in ST

call    fdivrp_sti_st                   ; do actual divide

pop     eax

ret

spill_fpstack_m16ir:

sub     esp, SPILL_SIZE                 ; make temp space

fstp    tbyte ptr[esp ]                 ; save user's ST(1)

fild    word ptr[esp + SPILL_MEM_OPERAND] ; load m16int

call    fdivrp_sti_st                   ; do actual divide

fld     tbyte ptr[esp]                  ; restore user's (TOS-1)

fincstp                                 ; restore FP stack

add     esp, SPILL_SIZE

pop     eax

ret

_fdivr_m16i     ENDP

;;; FDIVR_M32I - FDIVR m32int FIX

;;

;;      Input : Value of the m32int in the top of STACK

;;

;;      Output: Result of FDIVR in ST

PUBLIC  _fdivr_m32i

_fdivr_m32i     PROC    NEAR

push    eax                             ; save eax

call    fdiv_tos_valid                  ; check if (TOS-1) is empty

cmp     eax, 0                          ; is TOS-1 not empty?

je      spill_fpstack_m32ir             ; spill TOS-1 reg

fild    dword ptr[esp + MEM_OPERAND]    ; load m32int in ST

call    fdivrp_sti_st                   ; do actual divide

pop     eax

ret

spill_fpstack_m32ir:

sub     esp, SPILL_SIZE                 ; make temp space

fstp    tbyte ptr[esp ]                 ; save user's ST(1)

fild    dword ptr[esp + SPILL_MEM_OPERAND] ; load m32int

call    fdivrp_sti_st                   ; do actual divide

fld     tbyte ptr[esp]                  ; restore user's (TOS-1)

fincstp                                 ; restore FP stack

add     esp, SPILL_SIZE

pop     eax

ret

_fdivr_m32i     ENDP

fdiv_tos_valid  PROC    NEAR

fdecstp                         ; make (ST-1) TOS

fxam                            ; examine ST(-1)

fnstsw  ax                      ; get status word

and     eax, C023MASK           ; isolate C0, C2, and C3

cmp     eax, EMPTY_MASK         ; is ST(-1) empty?

je      st_1_empty              ; ST(-1) is empty

xor     eax, eax                ; return 0 for spill

ret

st_1_empty:

fincstp                         ; restore FP stack

ret                             ; return non zero value

fdiv_tos_valid  ENDP

CODE32  ENDS

end

wrap32.asm:

-----------

; static char sccs_id[] = "@(#)wrap32.asm       1.1 12/22/94 12:49:15";

;

; This code is being published by Intel to users of the Pentium(tm)

; processor.  Recipients are authorized to copy, modify, compile, use and

; distribute the code.

;

; Intel makes no warranty of any kind with regard to this code, including

; but not limited to, implied warranties or merchantability and fitness for

; a particular purpose. Intel assumes no responsibility for any errors that

; may appear in this code.

;

; No patent licenses are granted, express or implied.

;

;

;  The following code is a PRELIMINARY IMPLEMENTATION of a

;  software patch for the FPATAN instruction replacement.

;

;

.386

.387

DATA32  SEGMENT DWORD USE32 PUBLIC 'DATA'

FPU_STATE       STRUC

CONTROL_WORD    DW      ?

reserved_1      DW      ?

STATUS_WORD     DW      ?

reserved_2      DW      ?

TAG_WORD        DW      ?

reserved_3      DW      ?

IP_OFFSET       DD      ?

CS_SLCT         DW      ?

OPCODE          DW      ?

DATA_OFFSET     DD      ?

OPERAND_SLCT    DW      ?

reserved_4      DW      ?

REGISTER_0      DT      ?

REGISTER_1      DT      ?

REGISTER_2      DT      ?

REGISTER_3      DT      ?

REGISTER_4      DT      ?

REGISTER_5      DT      ?

REGISTER_6      DT      ?

REGISTER_7      DT      ?

SAVE_REG_0      DT      ?

FPU_STATE       ENDS

ENV_SIZE        EQU     118

DATA32  ENDS

CODE32  SEGMENT   DWORD USE32 PUBLIC 'CODE'

assume cs:CODE32, ds:FLAT, es:FLAT, ss:nothing

public  fpatan_wrap

;

;  PRELIMINARY VERSION for FPATAN instruction replacement

;

EXTRN   fatan_chk:NEAR

fpatan_wrap     PROC    NEAR

push    eax

sub     esp, ENV_SIZE

fstp    tbyte ptr [esp].SAVE_REG_0

fnsave  [esp]

fld     tbyte ptr [esp].REGISTER_0

fld     tbyte ptr [esp].SAVE_REG_0

call    fatan_chk

fnstsw  ax

fstp    tbyte ptr [esp].REGISTER_0

and     ax, 0ffh

or      ax, [esp].STATUS_WORD

mov     [esp].STATUS_WORD, ax

frstor  [esp]

add     esp, ENV_SIZE

pop     eax

ret

fpatan_wrap     ENDP

CODE32  ENDS

end

------------------------- Note headers follow --------------------------

From kjs@gomez.sc.intel.com  Sat Jan 14 04:26:26 1995

Return-Path: <kjs@gomez.sc.intel.com>

Received: from vkhdib01.hda.hydro.com by sk08ts.hda.hydro.com (IBM OS/2 SENDMAIL VERSION 1.3.2)/1.2)

id AA9980; Sat, 14 Jan 95 04:26:26 +0100

Received: from hermes.intel.com by hda.hydro.com with SMTP

id <AAvkhdib0124258>; Sat, 14 Jan 1995 04:26:25 +0100

Received: from gomez.sc.intel.com by hermes.intel.com (5.65/10.0i); Fri, 13 Jan 95 19:25:58 -0800

Received: from morticia.sc.intel.com (morticia123.sc.intel.com) by gomez.sc.intel.com with SMTP id AA01811

(5.65c/IDA-1.4.4); Fri, 13 Jan 1995 19:25:29 -0800

Received: by morticia.sc.intel.com (10.1b); Fri, 13 Jan 1995 19:25:24 -0800